## LED Strobe/Wash/Blinder Light

User Manual

- Please read this manual carefully before use!

Thank you for your patronage. We are confident that our excellent products and service can satisfy you. For your own safety, please read this user manual carefully before installing the device.

In order to install , operate, and maintain the light correctly, we suggest that the installation and operation should be done by the verified technician with following the instruction strictly.

This moving laser bar’s output is amazing. It is well designed with professional aluminum alloy housing. It makes full use of the advanced electronic positioning technology, and with international standard DMX512 control system. It is ideal for TV show, disco, dancing hall, night club, stage...and some applications outdoor.

Parts inside the box:

| Item | Quantity | Unit |
| --- | --- | --- |
| Light fixture | 1 | pc |
| User Manuel | 1 | pc |
| Power Cable | 1 | pc |

Please check carefully that there is no damage caused by transportation. Please contact your local dealer if any problem.

Caution

Before leaving the factory, this unit has passed strict inspection, functions stable and perfect packing. If this unit is operated in any way different to the one described in this manual, the product may suffer damages and the guarantee becomes void. Furthermore, any other operation may lead to dangers like short circuit, burns, electric shock, crash etc. If any technology changes of this manual, company do not notice in further and have interpretation.

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Symbol Instruction On The Device

……………………………………………………………

…

CE Certificate

………

…

……………………………………………………

…

……………………………………………

…

…………………

…

Suitable directly installed on the common combustible materials devices

Only indoor using

Environmental protection

The minimum distance from lighted objects(meter)

Safety Instructions

This unit was in perfect condition before leaving factory. In order to maintain this condition and to ensure a safe operation, it is absolutely necessary for the user to follow the safety instructions and warning notes written in this user manual.

►Unpack and check carefully there is no transportation damage before using the unit.

►Disconnect main power before replacement or servicing.

►The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent

surfaces. Be sure that no ventilation slots are blocked.

►Maximum ambient temperature is Ta: 40 degrees C. DO NOT operate it where the temperature is

higher than this.

►In the event of a serious operating problem, stop using the unit immediately. Never try to repair the

unit by yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please

contact the nearest authorized technical assistance center. Always use the same type spare parts.

►DO NOT connect any dimmer.

►DO NOT touch any cables during operation as high voltage may cause electric shock.

►To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or moisture.

►After unit working for 8 hours continuously, let the unit to take appropriate rest. It can reduce some damage and good for lifetime.

►Laser radiation avoids body exposure, especially for eye, it’s easy to cause blindness. Please keep the

safe distance at 1 meters above.

►To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or moisture.

►The housing and lenses must be replaced if they are visibly damaged.

CAUTION!

Please consider the GB7000.15/EN60598-2-17 and the other respective national norms during

The installation. The installation must only be carried out by a qualified person.

Installation Instruction

- This unit can not be installed on the surface of flammable materials. Please ensure the unit is at least 0.5m away from flammable and explosive materials.

- The installation must always be secured with a secondary safety e.g. an appropriate safety rope.Never stand directly below the device when mounting, removing or servicing the fixture.

- The applicable temperature for the lighting is between -10°C to 45°C. Do not use the lighting under or above the temperature.

- Before installation, must ensure installation bracket, handle or truss for this light can hold 10 times of the fixture weight for 1 hour without any harming deformation.

- The operator has to make sure the safety relating and machine technical installations are approved by an expert before taking the device into operation for the first time.

- These installations have to be approved by a skilled person once a year.

- Overhead mounting requires extensive experience, including others calculating working load limits, installation material being used, and periodic safety inspection of all installation material and the device. If you lack these qualifications, do not attempt the installation yourself. Improper installation can result in bodily injury.

Insert the quick-lock fasteners of the first holder into the respective holes on the bottom of the device.Tighten the quick-lock fasteners fully clockwise.

Make the finished installed mounting bracket tighten on the fixed bracket.

Pull the safety rope through the holes on the bottom of the base and over the trussing system or other safe fixation spot.

Note:This step is quite important to ensure that fixture will not drop out by the damage of clamp.

Production specification

Voltage：AC100 ~240V  50/60Hz，

Power consumption: 240W

LED: 960pcs SMD5050 RGB LED

Control mode ：Master-Slave/Auto/DMX512/Sound/ RDM

DMX channel：3CH/9CH/24CH

Dimming: 32bit, 0~100% linear dimming

Features: 8-segment horse racing + Wash + flashing

Working Temp.:-30°~ 50°

Strobe: 1~30-Hz；

Case: metal, black

Connection: DMX input/output,Powercon input/output

IP rating: IP20

Display Operation

| MENU | Select function |
| --- | --- |
| UP | Parameters increment |
| DOWN | Decreasing parameters |
| ENTER | Confirm and save |

MENU operation

After powering on, press the menu button, and the menu function table will appear in sequence; the up or down button to modify the function parameters, the confirm button to save the current functions and parameters (with power-off memory after saving).

Menu function table:

| A001 |  | A512 | Modify the address code (A001~A512) up or down, and save with the confirm key. The default is A001. |
| --- | --- | --- | --- |
| CH03 |  | CH24 | Switch between CH03, CH09, CH24 three channels up or down, confirm key to save, default CH09. |
| FF00 |  | FF99 | Gradient, modify the gradual speed up or down (FF00~FF99), confirm to save, the default is FF10. |
| EE00 |  | EE99 | Pulse change, modify the pulse change speed up or down (EE00~EE99), confirm to save, default EE10. |
| P000 |  | P241 | 242 kinds of built-in effects (P000~P241), switch the built-in effects up or down, save by the ENTER key, and jump to P000 by default. |
| S000 |  | S255 | Modify the operating speed of the built-in effect (S000~S255) up or down, and save with the ENTER key. The default is S010. |
| Soud |  | Soud | Sound control mode |
| R255 |  | R000 | Modify the brightness of RED up or down (R000~R255), and save with the ENTER key. The default is R255. |
| G255 |  | G000 | Modify the brightness of GREEN up or down (R000~R255), and save with the ENTER key. The default is G255. |
| B255 |  | B000 | Modify the brightness of BLUE up or down (R000~R255), and save with the ENTER key. The default is B255. |
| T000 |  |  | Display temperature. For example, T045 means that the current fixture temperature is 45℃; if 10K thermistor is not installed, T000 is displayed. |

Master / Slave control

Two or more of the same lights are connected with DMX three-core signal line, the light is set to any address code of A001~A512, any one is set as the master, and the other lamps are the slaves, and all the slaves display screens do not flash; use the master to fade , Pulse change, jump change, sound control, and auto effects, all slaves will synchronize gradual change, pulse change, jump, sound control, and auto effects.

Special attention:

1. Only one host can be set for a group of lights. If there are more than one host, all lamps will flash randomly and out of sync.

2. All lights can be the master/slave mode when the DMX512 console is turned off.

Factory setting

When any address code is A001~A512, press the menu button for 3 seconds to enter the factory setting. Factory settings are mainly the functions of each output power of the lamp, fan setting mode, setting temperature protection point, and sending parameters. Any mode in the factory setting can be exited by pressing the menu button for 3 seconds.

Factory setting table:

| R255 |  | R032 | Modify the RED (R032-R255) up or down, confirm to save.The default is R240. |
| --- | --- | --- | --- |
| G255 |  | G032 | Modify the GREEN (G032-G255) up or down, confirm to save. The default is G240. |
| B255 |  | B032 | Modify the BLUE (b032-B255) up or down, confirm to save. The default is B240. |
| FAN0 |  | FAN1 | Fan setting: FAN0 lamp bead lights up to start the fan, FAN1 reaches the set temperature protection point to start the fan, confirm to save. |
| T040 |  | T105 | Set the temperature protection point, modify the parameter up or down (40℃~105℃), confirm to save, the default is T060. |
| Send |  | Send | Send the factory setting parameters of the machine up or down to all other lights connected in parallel with the three-core signal line; confirm the sending of the parameters and press the menu button for 3 seconds to exit. If the parameters are denied, press the ENTER button to cancel the sending. |

DMX512 Channel

- 3-Channel table

| Channel | Value | Function |
| --- | --- | --- |
| 1 | 000-255 | Red linear dimming |
| 2 | 000-255 | Green linear dimming |
| 3 | 000-255 | Blue linear dimming |

- 9-Channel table

| Channel | Value | Function |
| --- | --- | --- |
| 1 | 000-255 | General Dimming |
| 2 | 000-255 | Red linear dimming |
| 3 | 000-255 | Green linear dimming |
| 4 | 000-255 | Blue linear dimming |
| 5 | 000-255 | Strobe |
| 6 | 000-255 | Mode（See: Mode effect 1） |
| 7 | 000-255 | Mode（See: Mode effect 2） |
| 8 | 000-255 | Mode（See: Mode effect 3） |
| 9 | 000-255 | Speed |

- Channel table

| Channel | value | Function |
| --- | --- | --- |
| 1 | 000-255 | R segment-1 linear dimming |
| 2 | 000-255 | G segment-1 linear dimming |
| 3 | 000-255 | B segment-1 linear dimming |
| … | … | … … |
| 22 | 000-255 | R segment-8 linear dimming |
| 23 | 000-255 | G segment-8 linear dimming |
| 24 | 000-255 | B segment-8 linear dimming |

Mode effect 1 (mode number 2-82, changeable color by push-and-pull RGB)

| Value | Mode no. | effect |
| --- | --- | --- |
| 0-2 | 0 | No function |
| 3-5 | 1 | Jumping |
| 6-8 | 2 | One segement Red LED clockwise riding |
| 9-11 | 3 | One segement Green LED clockwise riding |
| 12-14 | 4 | One segement Blue LED clockwise riding |
| 15-17 | 5 | One segement RG LED clockwise riding |
| 18-20 | 6 | One segement RB LED clockwise riding |
| 21-23 | 7 | One segement GB LED clockwise riding。 |
| 24-26 | 8 | One segement RGB LED clockwise riding |
| 27-29 | 9 | Macro mode number 2-8 cycle |
| 30-32 | 10 | One segement Red LED anti-clockwise riding |
| 33-35 | 11 | One segement Green LED anti-clockwise riding |
| 36-38 | 12 | One segement Blue LED anti-clockwise riding |
| 39-41 | 13 | One segement RG LED anti-clockwise riding |
| 42-44 | 14 | One segement RB LED anti-clockwise riding |
| 45-47 | 15 | One segement GB LED anti-clockwise riding。 |
| 48-50 | 16 | One segement RGB LED anti-clockwise riding |
| 51-53 | 17 | Macro mode number 10-16 cycle |
| 54-56 | 18 | two segement Red LED anti-clockwise riding |
| 57-59 | 19 | two segement Green LED anti-clockwise riding |
| 60-62 | 20 | two segement Blue LED anti-clockwise riding |
| 63-65 | 21 | two segement RG LED anti-clockwise riding |
| 66-68 | 22 | two segement RB LED anti-clockwise riding |
| 69-71 | 23 | two segement GB LED anti-clockwise riding。 |
| 72-74 | 24 | two segement RGB LED anti-clockwise riding |
| 75-77 | 25 | Macro mode number 18-24 cycle |
| 78-80 | 26 | two segement Red LED clockwise riding |
| 81-83 | 27 | two segement Green LED clockwise riding |
| 84-86 | 28 | two segement Blue LED clockwise riding |
| 87-89 | 29 | two segement RG LED clockwise riding |
| 90-92 | 30 | two segement RB LED clockwise riding |
| 93-95 | 31 | two segement GB LED clockwise riding。 |
| 96-98 | 32 | two segement RGB LED clockwise riding |
| 99-101 | 33 | Macro mode number 26-32 cycle |
| 102-104 | 34 | Two segement Red LED stacking |
| 105-107 | 35 | Two segement Green LED stacking |
| 108-110 | 36 | Two segement Blue LED stacking |
| 111-113 | 37 | Two segement RG LED stacking |
| 114-116 | 38 | Two segement RB LED stacking |
| 117-119 | 39 | Two segement GB LED stacking |
| 120-122 | 40 | Two segement RGB LED stacking |
| 123-125 | 41 | Macro mode number 34-40 cycle |
| 126-128 | 42 | One segement Red LED stacking |
| 129-131 | 43 | One segement Green LED stacking |
| 132-134 | 44 | One segement Blue LED stacking |
| 135-137 | 45 | One segement RG LED stacking |
| 138-140 | 46 | One segement RB LED stacking |
| 141-143 | 47 | One segement GB LED stacking |
| 144-146 | 48 | One segement RGB LED stacking |
| 147-149 | 49 | Macro mode number 42-48 cycle |
| 150-152 | 50 | One segement Red LED and One segement Green LED anti-clockwise riding |
| 153-155 | 51 | One segement Blue LED and One segement Green LED anti-clockwise riding |
| 156-158 | 52 | One segement RG LED and One segement Blue LED anti-clockwise riding |
| 159-161 | 53 | One segement RG LED and One segement RB LED anti-clockwise riding |
| 162-164 | 54 | One segement RB LED and One segement GB LED anti-clockwise riding |
| 165-167 | 55 | One segement RGB LED and One segement GB LED anti-clockwise riding |
| 168-170 | 56 | One segement Red LED and One segement RGB LED anti-clockwise riding |
| 171-173 | 57 | Macro mode number 50-56 cycle |
| 174-176 | 58 | Two segement Red LED anti-clockwise refresh |
| 177-179 | 59 | Two segement Green LED anti-clockwise refresh |
| 180-182 | 60 | Two segement Blue LED anti-clockwise refresh |
| 183-185 | 61 | Two segement RG LED anti-clockwise refresh |
| 186-188 | 62 | Two segement RB LED anti-clockwise refresh |
| 189-191 | 63 | Two segement GB LED anti-clockwise refresh |
| 192-194 | 64 | Two segement RGB LED anti-clockwise refresh |
| 195-197 | 65 | Macro mode number 58-64 cycle |
| 198-200 | 66 | Two segement Red LED clockwise refresh |
| 201-203 | 67 | Two segement Green LED clockwise refresh |
| 204-206 | 68 | Two segement Blue LED clockwise refresh |
| 207-209 | 69 | Two segement RG LED clockwise refresh |
| 210-212 | 70 | Two segement RB LED clockwise refresh |
| 213-215 | 71 | Two segement GB LED clockwise refresh |
| 216-218 | 72 | Two segement RGB LED clockwise refresh |
| 219-221 | 73 | Macro mode number 66-72 cycle |
| 222-224 | 74 | Two segement Red LED anti-clockwise refresh |
| 225-227 | 75 | Two segement Green LED anti-clockwise refresh |
| 228-230 | 76 | Two segement Blue LED anti-clockwise refresh |
| 231-233 | 77 | Two segement RG LED anti-clockwise refresh |
| 234-236 | 78 | Two segement RB LED anti-clockwise refresh |
| 237-239 | 79 | Two segement GB LED anti-clockwise refresh |
| 240-242 | 80 | Two segement RGB LED anti-clockwise refresh |
| 243-245 | 81 | Macro mode number 74-80 cycle |
| 246-248 | 82 | 7 colors clockwise refresh |
| 249-251 | 83 | 7 colors clockwise riding |
| 252-254 | 84 | Mode number 2-83 cycle |
| 255 | 85 | Sound |

Mode effect 2 (mode number 2-82, changeable color by push-and-pull RGB)

| Value | Mode no. | effect |
| --- | --- | --- |
| 0-2 | 0 | No effect |
| 3-5 | 1 | Red all on |
| 6-8 | 2 | Green all on |
| 9-11 | 3 | Blue all on |
| 12-14 | 4 | Red+Green all on |
| 15-17 | 5 | Red+Blue all on |
| 18-20 | 6 | Green+Blue all on |
| 21-23 | 7 | Red+Green+Blue all on |
| 24-26 | 8 | Macro mode number 1-7 cycle |
| 27-29 | 9 | Pulse change |
| 30-32 | 10 | Jumping |
| 33-35 | 11 | One strip Red LED anti-clockwise riding |
| 36-38 | 12 | One strip Green LED anti-clockwise riding |
| 39-41 | 13 | One strip Blue LED anti-clockwise riding |
| 42-44 | 14 | One strip RG LED anti-clockwise riding |
| 45-47 | 15 | One strip RB LED anti-clockwise riding |
| 48-50 | 16 | One strip GB LED anti-clockwise riding。 |
| 51-53 | 17 | One strip RGB LED anti-clockwise riding |
| 54-56 | 18 | Macro mode number 11-17 cycle |
| 57-59 | 19 | One strip Red LED clockwise riding |
| 60-62 | 20 | One strip Green LED clockwise riding |
| 63-65 | 21 | One strip Blue LED clockwise riding |
| 66-68 | 22 | One strip RG LED clockwise riding |
| 69-71 | 23 | One strip RB LED clockwise riding |
| 72-74 | 24 | One strip GB LED clockwise riding |
| 75-77 | 25 | One strip RGB LED clockwise riding |
| 78-80 | 26 | Macro mode number 19-25 cycle |
| 81-83 | 27 | Two strip Red riding back and forth. |
| 84-86 | 28 | Two strip Green riding back and forth. |
| 87-89 | 29 | Two strip Blue riding back and forth. |
| 90-92 | 30 | Two strip RG riding back and forth. |
| 93-95 | 31 | Two strip RB riding back and forth. |
| 96-98 | 32 | Two strip GB riding back and forth. |
| 99-101 | 33 | Two strip RGB riding back and forth. |
| 102-104 | 34 | Macro mode number 27-33 cycle |
| 105-107 | 35 | Two strip Red colliding back and forth. |
| 108-110 | 36 | Two strip Green colliding back and forth. |
| 111-113 | 37 | Two strip Blue colliding back and forth. |
| 114-116 | 38 | Two strip RG colliding back and forth. |
| 117-119 | 39 | Two strip RB colliding back and forth. |
| 120-122 | 40 | Two strip GB colliding back and forth. |
| 123-125 | 41 | Two strip RGB colliding back and forth. |
| 126-128 | 42 | Macro mode number 35-41 cycle |
| 129-131 | 43 | One strip Red LED riding |
| 132-134 | 44 | One strip Green LED riding |
| 135-137 | 45 | One strip Blue LED riding |
| 138-140 | 46 | One strip RG LED riding |
| 141-143 | 47 | One strip RB LED riding |
| 144-146 | 48 | One strip GB LED riding |
| 147-149 | 49 | One strip RGB LED riding |
| 150-152 | 50 | Macro mode number 43-49 cycle |
| 153-155 | 51 | One strip Red LED stacking |
| 156-158 | 52 | One strip Green LED stacking |
| 159-161 | 53 | One strip Blue LED stacking |
| 162-164 | 54 | One strip RG LED stacking |
| 165-167 | 55 | One strip RB LED stacking |
| 168-170 | 56 | One strip GB LED stacking |
| 171-173 | 57 | One strip RGB LED stacking |
| 174-176 | 58 | Macro mode number 51-57cycle |
| 177-179 | 59 | One strip Red LED and One strip Green LED anti-clockwise riding |
| 180-182 | 60 | One strip Blue LED and One strip Green LED anti-clockwise riding |
| 183-185 | 61 | One strip RG LED and One strip Blue LED anti-clockwise riding |
| 186-188 | 62 | One strip RG LED and One strip RB LED anti-clockwise riding |
| 189-191 | 63 | One strip RB LED and One strip GB LED anti-clockwise riding |
| 192-194 | 64 | One strip RGB LED and One strip GB LED anti-clockwise riding |
| 195-197 | 65 | One strip Red LED and One strip RGB LED anti-clockwise riding |
| 198-200 | 66 | Macro mode number 50-56 cycle |
| 201-203 | 67 | One strip Red LED refresh from left to right. |
| 204-206 | 68 | One strip Green LED refresh from left to right. |
| 207-209 | 69 | One strip Blue LED refresh from left to right. |
| 210-212 | 70 | One strip RG LED refresh from left to right. |
| 213-215 | 71 | One strip RB LED refresh from left to right. |
| 216-218 | 72 | One strip GB LED refresh from left to right. |
| 219-221 | 73 | One strip RGB LED refresh from left to right. |
| 222-224 | 74 | Macro mode number 67-73 cycle |
| 225-227 | 75 | One strip Red LED refresh from right to left. |
| 228-230 | 76 | One strip Green LED refresh from right to left. |
| 231-233 | 77 | One strip Blue LED refresh from right to left. |
| 234-236 | 78 | One strip RG LED refresh from right to left. |
| 237-239 | 79 | One strip RB LED refresh from right to left. |
| 240-242 | 80 | One strip GB LED refresh from right to left. |
| 243-245 | 81 | One strip RGB LED refresh from right to left. |
| 246-248 | 82 | Macro mode number 75-81 cycle |
| 249-251 | 83 | 7 colors clockwise riding |
| 252-254 | 84 | Mode number 2-83 cycle |
| 255 | 85 | Sound |

Mode effect 3 (mode number 2-82, changeable color by push-and-pull RGB)

| Value | Mode no. | effect |
| --- | --- | --- |
| 0-2 | 0 | No effect |
| 3-5 | 1 | Jumping |
| 6-8 | 2 | two segement Red LED clockwise running |
| 9-11 | 3 | two segement Green LED clockwise running |
| 12-14 | 4 | two segement Blue LED clockwise running |
| 15-17 | 5 | two segement RG LED clockwise running |
| 18-20 | 6 | two segement RB LED clockwise running |
| 21-23 | 7 | two segement GB LED clockwise running |
| 24-26 | 8 | two segement RGB LED clockwise running |
| 27-29 | 9 | Macro mode number 2-8 cycle |
| 30-32 | 10 | two segement Red LED anti-clockwise running |
| 33-35 | 11 | two segement Green LED anti-clockwise running |
| 36-38 | 12 | two segement Blue LED anti-clockwise running |
| 39-41 | 13 | two segement RG LED anti-clockwise running |
| 42-44 | 14 | two segement RB LED anti-clockwise running |
| 45-47 | 15 | two segement GB LED anti-clockwise running |
| 48-50 | 16 | two segement RGB LED anti-clockwise running |
| 51-53 | 17 | Macro mode number 10-16cycle |
| 54-56 | 18 | two segement Red LED reverse running |
| 57-59 | 19 | two segement Green LED reverse running |
| 60-62 | 20 | two segement Blue LED reverse running |
| 63-65 | 21 | two segement RG LED reverse running |
| 66-68 | 22 | two segement RB LED reverse running |
| 69-71 | 23 | two segement GB LED reverse running |
| 72-74 | 24 | two segement RGB LED reverse running |
| 75-77 | 25 | Macro mode number 18-24cycle |
| 78-80 | 26 | two segement Red LED reverse running |
| 81-83 | 27 | two segement Green LED reverse running |
| 84-86 | 28 | two segement Blue LED reverse running |
| 87-89 | 29 | two segement RG LED reverse running |
| 90-92 | 30 | two segement RB LED reverse running |
| 93-95 | 31 | two segement GB LED reverse running |
| 96-98 | 32 | two segement RGB LED reverse running |
| 99-101 | 33 | Macro mode number 26-32 cycle |
| 102-104 | 34 | two segement Red LED connected and clockwise running |
| 105-107 | 35 | two segement Green LED connected and clockwise running |
| 108-110 | 36 | two segement Blue LED connected and clockwise running |
| 111-113 | 37 | two segement RG LED connected and clockwise running |
| 114-116 | 38 | two segement RB LED connected and clockwise running |
| 117-119 | 39 | two segement GB LED connected and clockwise running |
| 120-122 | 40 | two segement RGB LED connected and clockwise running |
| 123-125 | 41 | Macro mode number 34-40 cycle |
| 126-128 | 42 | two segement Red LED connected and anti-clockwise running |
| 129-131 | 43 | two segement Green LED connected and anti-clockwise running |
| 132-134 | 44 | two segement Blue LED connected and anti-clockwise running |
| 135-137 | 45 | two segement RG LED connected and anti-clockwise running |
| 138-140 | 46 | two segement RB LED connected and anti-clockwise running |
| 141-143 | 47 | two segement GB LED connected and anti-clockwise running |
| 144-146 | 48 | two segement RGB LED connected and anti-clockwise running |
| 147-149 | 49 | Macro mode number 42-48 cycle |
| 150-152 | 50 | Four segement Red running back and forth. |
| 153-155 | 51 | Four segement Green running back and forth. |
| 156-158 | 52 | Four segement Blue running back and forth. |
| 159-161 | 53 | Four segement RG running back and forth. |
| 162-164 | 54 | Four segement RB running back and forth. |
| 165-167 | 55 | Four segement GB running back and forth. |
| 168-170 | 56 | Four segement RGB running back and forth. |
| 171-173 | 57 | Macro mode number 50-56 cycle |
| 174-176 | 58 | Four segement Red running back and forth. |
| 177-179 | 59 | Four segement Green running back and forth. |
| 180-182 | 60 | Four segement Blue running back and forth. |
| 183-185 | 61 | Four segement RG running back and forth. |
| 186-188 | 62 | Four segement RB running back and forth. |
| 189-191 | 63 | Four segement GB running back and forth. |
| 192-194 | 64 | Four segement RGB running back and forth. |
| 195-197 | 65 | Macro mode number 58-64 cycle |
| 198-200 | 66 | One segement Red clockwise running in the middle |
| 201-203 | 67 | One segement Green clockwise running in the middle |
| 204-206 | 68 | One segement Blue clockwise running in the middle |
| 207-209 | 69 | One segement RG clockwise running in the middle |
| 210-212 | 70 | One segement RB clockwise running in the middle |
| 213-215 | 71 | One segement GB clockwise running in the middle |
| 216-218 | 72 | One segement RGB rclockwise running in the middle |
| 219-221 | 73 | Macro mode number 66-72 cycle |
| 222-224 | 74 | One segement Red anti-clockwise running in the middle |
| 225-227 | 75 | One segement Green anti-clockwise running in the middle |
| 228-230 | 76 | One segement Blue anti-clockwise running in the middle |
| 231-233 | 77 | One segement RG anti-clockwise running in the middle |
| 234-236 | 78 | One segement RB anti-clockwise running in the middle |
| 237-239 | 79 | One segement GB anti-clockwise running in the middle |
| 240-242 | 80 | One segement RGB anti-clockwise running in the middle |
| 243-245 | 81 | Macro mode number 74-80cycle |
| 246-248 | 82 | 7-color selection，divided to 2 segement different colors,reverse runing in circle |
| 249-251 | 83 | 7 colors clockwise riding |
| 252-254 | 84 | Mode number 2-83 cycle |
| 255 | 85 | Sound |

DMX-512 connection

Connect the provided XLR cable to the male 3-pin XLR output of your controller and the other side to the female 3-pin XLR input of the light.You can chain multiple lights together through serial linking

- The cable needed should be two core, screened cable with XLR input and output connectors. Please refer to the diagram below.

XLR Mounting socket

XLR Mounting plug

DMX address setting

Each fixture must have a specific starting address. When receiving signal transmission, fixture will receive channel control signals from DMX controller through this starting address.

According to different requirements or easy to use, the user can set many fixtures to the same address, also can set up a separately address for every device.

If more than one fixtures have the same address, all of them receive the same DMX channel signal from DMX controller. All connected fixtures are jointly controlled, DMX controller can't control a device separately.

If each device has a different address, each device will be separate and accept separate DMX512 control signal through the separate DMX address, so that more convenient to control one device separately. this method must be based on channel quantity of device to determine a starting address.

Cleaning and maintenance

- Cleaning should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

Cut off power before cleaning and maintenance!

CAUTION

Attention

Do not see directly light source when open the light, and make sure power off before do any installation or maintenance.

Warranty Card

We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product needs maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：