LED Par light

User Manual

12pcs RGBW 5in1 LED Par light

USER MANUAL

Product Specification

- 5/9 DMX512 channels

- Sound-Active/Auto-Play/DMX 512/Master-Slave

- LED display

- 3-pins XLR input/output

- Power supply: AC100V-240V  50/60Hz

- LED: 12pcs RGBW+UV LED

- Protection Rate: IP20

LED display

Operation instruction: Press button MENU will cycle display 10 different function effects, the front two value of LED digit means the current function( see function map). The end two value means functions’ address and speed. Press DOWN or UP can adjust different value, press ENTER to enter.

LED digit display menu instruction

| NO | Display | Value | Function |
| --- | --- | --- | --- |
| 1 | d001 | 001- 512 | DMX512 5DMX channel address setting |
| 2 | A001 | 001- 512 | DMX512 9DMX channel address setting |
| 3 | r255 | 000 - 255 | red dimmer from 0-100% |
| 4 | G255 | 000 - 255 | green dimmer from 0-100% |
| 5 | b255 | 000 - 255 | blue dimmer from 0-100% |
| 6 | u255 | 000 - 255 | white dimmer from 0-100% |
| 7 | Y255 | 000 - 255 | UV dimmer from 0-100% |
| 8 | FH99 | 01 - 99 | strobe slow to fast |
| 9 | CL01 | 01-08 | 8 color effect |
| 10 | CC99 | 01 - 99 | jump color change slow to fast |
| 11 | DE99 | 01 - 99 | color fading slow to fast |
| 12 | CP99 | 01 - 99 | pulse change slow to fast |
| 13 | SU01 | 01-09 | sound activated |

DMX 512 Channel

| Channel |  | Value | Function |
| --- | --- | --- | --- |
| 5CH | 9CH |  |  |
|  | CH1 | 0-255 | Master dimmer |
| CH1 | CH2 | 0-255 | Red dimmer |
| CH2 | CH3 | 0-255 | Green dimmer |
| CH3 | CH4 | 0-255 | Blue dimmer |
| CH4 | CH5 | 0-255 | White dimmer |
| CH5 | CH6 | 0-255 | UV dimmer |
|  | CH7 | 0-255 | Strobe slow to fast |
|  | CH8 | 0-50 | DMX dimmer |
|  |  | 51-100 | 8 color effect |
|  |  | 101-150 | jump color change |
|  |  | 151-200 | color fading |
|  |  | 201-250 | pulse change |
|  |  | 251-255 | sound activated |
|  | CH9 | 0-255 | speed slow to fast |

Safety instruction

The electric connection must carry out by qualified person.

Make sure that the available voltage is not higher than stated at the end of this manual.

Make sure the power cord is never crimped or damaged by sharp edges. If this would be the case, replacement of the cable must be done by an authorized dealer.

If the external flexible cable or cord of this luminaire is damaged, it shall be exclusively replaced by the manufacturer or his service agent or a similar qualified person in order to avoid a hazard.

Please cut off the power if don't use the fixture or want to clean, maintenance.