LED BEAM MOVING HEAD

USER MANUAL

Please read and keep this manual carefully for further use!

Thank you for buying our professional Beam moving head.

This user manual includes important information for installation and operation, so please read this user manual carefully before installation and operation, and keep this user manual well for further use.

In order to install , operate, and maintain the lighting safety and correctly. We suggest that the installation and operation should be done by the verified technician and follow the instruction strictly.

Spare part includes:

| Name | QTY | Unit |
| --- | --- | --- |
| Fixture | 1 | PC |
| User manual | 1 | PC |
| Power cable | 1 | PC |
| Bracket | 2 | PCS |
| DMX signal cable | 1 | PC |
| Safety rope | 1 | PC |

Please check carefully that there is no damage caused by transportation. Should there be any, consult your local dealer.

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Silkscreen symbol on light

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Installation

► This fixture is not suitable for direct installation on combustible material surface, please keep the fixture surface at least 0.5m from any combustible material.

► If you use the quick lock cam in hanging up the fixture, please make sure the quick lock fasteners turned in the quick lock holes correctly.

►The applicable temperature for the fixture is between -25°C to 45°C. Do not use the lighting under or above this temperature range.

►The truss for hanging the fixture should be able to hold 10 times weight of this fixture and no deformation after 1 hour hang.

►Please don’t stand under the fixture when install, uninstall, move or repair the fixture.

►Please ensure the light is connected correctly and invite professional technician to check and confirm the electrical data before installation.

►Please invite professional technician to check the fixture and installation each year.

Rigging：

Clamp

Bracket

Safety rope

Quick lock

Insert the quick-lock fasteners of the first holder into the respective holes on the bottom of the device.Tighten the quick-lock fasteners fully clockwise.

Fix the clamp on truss or other fixture bracket.

Pull the safety rope through the holes on the bottom of the base and fasten it on the truss or other fixing point.

Note:This step is quite important to ensure that fixture will not drop down for the clamp damage.

Product specification

1. Voltage：100-240V，50/60HZ；

2. Power Consumption：200W；

3. Lamp:200W LED

4.Fuse:6.3A/250V，￠5*20

5.DMX Channel: 19CH

6.Operation mode: master-slave/DMX/Auto

7. Focus: Yes

8. Fixed gobo: 12 gobos+open+ 5 glass gobos

9. Electronic strobe: 25Hz

10. Color: 14 colors+open

11. Prism 1: 8-facet

12. Prism 2: 8+16+32-facet

13. Pan: 540°

14. Tilt: 270°

15.Frost: Yes

16.Six-color effect: Yes

17. RDM: Yes

18. Standard beam angle: 2°

19. Display: LED LCD screen

Display and operation

| Touch button | Function |
| --- | --- |
| Up | Option, Edit |
| Down | Option, Edit |
| Left | Backtrack, Edit |
| Right | Backtrack, Edit |
| OK | Enter to edit , confirm |

Setting:

| Option | Description |  |
| --- | --- | --- |
| Operating mode | DMX | slave status: receiving DMX signals from the console or master |
|  | AUTO 1 | master state: auto and send DMX signal to the slave |
|  | AUTO 2 |  |
|  | Random |  |
|  | Sound |  |
| DMX address | 1~512 | Press the "OK" key to enter the editing state. |
| Channel mode | 19 CH | Standard 19-channel mode |
| X reverse | off |  |
|  | on |  |
| Y reverse | off |  |
|  | on |  |
| XY exchange | off |  |
|  | on | Exchange the channels of the XY axis (including fine-tuning) |
| XY encoder | on | Use the encoder (optocoupler) to judge the out-of-step and automatically correct the position |
|  | off | Does not use encoder (optocoupler) to correct position |
| DMX signal | hold | running in the original state |
|  | clear | the motor back to position, stop running |
| Lamp on | off | Reset directly after power on, the bulb will not on |
|  | on | lamp on after power on, the bulb will light up automatically, and it will be reset only after the bulb is on |
| Color wheel linear change | on | The color wheel changes linearly |
|  | off | Non-linear change of color wheel, half color change |
| Default settings |  | Press the "OK" button to see the confirmation dialog box, press the "OK" button again to restore the default settings |

19 DMX channel

| 19CH | Function | Value | Details |
| --- | --- | --- | --- |
| CH1 | Color wheel | 000-004 005 -009 010 - 014 015 - 019 020 - 024 025 - 029 030 - 034 035 - 039 040 - 044 045 - 049 050 - 054 055 - 059 060 - 064 065 - 069 070 - 074 075 - 079 080 - 084 085 - 089 090 - 094 095 - 099 100 -104 105 -109 110 -114 115 -119 120 -124 125 -129 130 -134 135 -139 140 -144 145 -149 150 - 200 201 - 255 | White White+Color 1 Color 1 Color 1+color 2 Color 2 Color 2+color 3 Color 3 Color 3+color 4 Color 4 Color 4+color 5 Color 5 Color 5+color 6 Color 6 Color 6+color 7 Color 7 Color 7+color 8 Color 8 Color 8+color 9 Color 9 Color 9+color 10 Color 10 Color 10+color 11 Color 11 Color 11+color 12 Color 12 Color 12+color 13 Color 13 Color 12+color 13 Color 14 Color 14+white light Reverse flow (from fast to slow) Forward flow (from slow to fast) |
| CH2 | dimmer | 0-255 | dimmer 0-100% |
| CH3 | strobe | 000-003 004-103 104-107 108-207 208-212 213-251 252-255 | Shutter closed Strobe from slow to fast Open the shutter → (controlled by the dimming channel) Pulse strobe from slow to fast Open the shutter → (controlled by the dimming channel) Random strobe from slow to fast Open the shutter → (controlled by the dimming channel) |

| CH4 | gobo wheel | 000 - 004 005 - 009 010 - 014 015 - 019 020 - 024 025 - 029 030 - 034 035 - 039 040 - 044 045 - 049 050 - 054 055 - 059 060 - 064 065 - 069 070 - 074 075 - 079 080 - 084 085 - 089 090 - 094 095 - 099 100 - 104 105 - 109 110 - 114 115 - 119 120 - 124 125 - 129 130 - 134 135 - 139 140 - 144 145 - 149 150 - 154 155 - 159 160 - 164 165 - 169 170 - 174 175 - 215 216 - 255 | gobo 1 gobo 2 gobo 3 gobo 4 gobo 5 gobo 6 gobo 7 gobo 8 gobo 9 gobo 10 gobo 11 gobo 12 gobo 13 gobo 14 gobo 15 gobo 16 gobo 17 gobo 18 gobo 2 shake slow to fast gobo 3 shake slow to fast gobo 4 shake slow to fast gobo 5 shake slow to fast gobo 6 shake slow to fast gobo 7 shake slow to fast gobo 8 shake slow to fast gobo 9 shake slow to fast gobo 10 shake slow to fast gobo 11 shake slow to fast gobo 12 shake slow to fast gobo 13 shake slow to fast gobo 14 shake slow to fast gobo 15 shake slow to fast gobo 16 shake slow to fast gobo 17 shake slow to fast gobo 18 shake slow to fast Reverse flow (from fast to slow) Forward flow (from slow to fast) |
| --- | --- | --- | --- |
| CH5 | Prism 1 | 000-127 128-255 | no effect prism 1 |
| CH6 | prism rotation | 000-127 128-190 191-192 193-255 | prism 1 adjust angle Reverse rotation fast to slow stop Forward rotation slow to fast |
| CH7 | Prism 2 | 000-127 128-255 | no effect prism 2 |
| CH8 | frost and six color effect | 000-127 128-191 192-255 | no function frost Six-color effect |
| CH9 | focus | 0-255 | focus 0-100% |
| CH10 | focus fine | 0-255 | focus fine |
| CH11 | pan | 0-255 | 0-540 degree |
| CH12 | pan fine | 0-255 | 0-1.2 degree |
| CH13 | tilt | 0-255 | 0-270 degree |
| CH14 | tilt fine | 0-255 | 0-1.2 degree |
| CH15 | XY speed | 0-255 | speed fast to slow |
| CH16 | Reset | 000-025 026-076 077-127 128-255 | no function small motors reset Pan/Tilt motors reset all motors reset |
| CH17 | LED strobe | 0-255 | Strobe from slow to fast |
| CH18 | LED effect | 0-255 |  |
| CH19 | LED speed | 0-255 | Speed from slow to fast |

Gobo wheel                           Color wheel

Six-color effect / Frost

DMX-512 connection

- Connect the provided XLR cable to the male 3-pin XLR output of your controller and the other side to the female 3-pin XLR input of the light.then connect the DMX signal to the first fixture to the 2nd fixture, after all fixtures are connected,

- The signal cable should be two - core with screened cable with XLR input and output connectors. Please refer to the diagram below

DMX terminator connection

If there are many lines or lights, it is recommended to use a DMX terminator to prevent DMX signal corruption, the DMX terminator is an simple XLR plug with a 120 Ω resistor connected between pins 2 and 3,which is then plugged into the output XLR socket of the last fixture in the chain.

3-Pins connection

Cleaning and maintenance

- Cleaning should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

Cut off power before cleaning and maintenance!

CAUTION

Attention

Do not see directly light source when open the light, and make sure power off before do any installation or maintenance.

Wires connection must be done professional person!

CAUTION

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

Common issues instruction

| Issues | Solve instruction |
| --- | --- |
| No power | Check power’s voltage |
| When power in, output light but the device cannot accept control signal | Check address setting and the control DMX cable connection well or not |
| The effect have light shadow | Check electronic focus DMX channel value, if it’s suitable distance |
| Light beam dim | The lamp maybe arrived end of using life, should change new lamp, and also check light source cleaning or not |

Note:  the maintenance should be done by the verified technician.

Warranty Card This product is made of high-brightness transistor. We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product need maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：