LED MOVING HEAD

USER MANUAL

Please read and keep this manual carefully for further use!

Thank you for buying our professional Beam moving head.

This user manual includes important information for installation and operation, so please read this user manual carefully before installation and operation, and keep this user manual well for further use.

In order to install , operate, and maintain the lighting safety and correctly. We suggest that the installation and operation should be done by the verified technician and follow the instruction strictly.

Spare part includes:

| Name | QTY | Unit |
| --- | --- | --- |
| Fixture | 1 | PC |
| User manual | 1 | PC |
| Power cable | 1 | PC |
| Bracket | 2 | PCS |
| DMX signal cable | 1 | PC |
| Safety rope | 1 | PC |

Please check carefully that there is no damage caused by transportation. Should there be any, consult your local dealer.

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Silkscreen symbol on light

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Installation

- ► This fixture is not suitable for direct installation on combustible material surface, please keep the fixture surface at least 0.5m from any combustible material.

- ► If you use the quick lock cam in hanging up the fixture, please make sure the quick lock fasteners turned in the quick lock holes correctly.

►The applicable temperature for the fixture is between -25°C to 45°C. Do not use the lighting under or above this temperature range.

►The truss for hanging the fixture should be able to hold 10 times weight of this fixture and no deformation after 1 hour hang.

►Please don’t stand under the fixture when install, uninstall, move or repair the fixture.

►Please ensure the light is connected correctly and invite professional technician to check and confirm the electrical data before installation.

►Please invite professional technician to check the fixture and installation each year.

Rigging：

Insert the quick-lock fasteners of the first holder into the respective holes on the bottom of the device.Tighten the quick-lock fasteners fully clockwise.

Fix the clamp on truss or other fixture bracket.

Pull the safety rope through the holes on the bottom of the base and fasten it on the truss or other fixing point.

Note:This step is quite important to ensure that fixture will not drop down for the clamp damage.

Product specification

Voltage：100-240V，50/60HZ；

Power Consumption：500W；

Lamp:400W White LED

DMX Channel: 24CH

Operation mode: master-slave/DMX/Auto/RDM

Electronic strobe: 25Hz

Dimming: 0-100% linear dimming

Fixed gobo: 11 gobos+open

Rotation gobo: 8 gobos+open

Color: 9 colors+open+CTO+CTB

Prism 1: 8-facet

Prism 2: 3-facet

Pan: 540°

Tilt: 270°

Frost: Yes

Focus: Yes

Zoom: Yes

CMY: Yes

Display: LED LCD touch screen

Display and operation

Panel:

| DMX | DMX signal indicator |
| --- | --- |
| AUDIO | Indicator light when voice activation is triggered |
| Up | menu up selection |
| Down | menu down selection |
| Left | Menu select left/back |
| Right | Menu select right |
| ENTER | confirmation button |
| Time on the left in the upper right corner of the screen | Total system usage time (display can be turned off in standard settings) |
| Time at the top right corner of the screen | The total time of this power-on (the display can be turned off in the standard setting) |
| Dot in the upper right corner of the screen | Green: OK, no errors |
|  | Red: There is an error (click "Information Menu/Error Information/View" to view the error |

System Menu description:

| 反向 | Reverse | Invert the screen display |
| --- | --- | --- |
| 退出 | Exit | Exit to the main interface |
| Language | 语言 | Simplified Chinese or English can be selected |
| 标准设置 | Standard | Address code and some common function settings |
| 高级设置 | Advanced | The main debugging parameter settings at the factory |
| 系统信息 | Info | Information display of errors, software versions, etc. |
| 手动编程 | Program | Manually control the operation, or edit the user program |
| 测试运行 | Perform | Auto-run or voice-activated built-in program, user program |
| 系统复位 | Reset | System reset to initial position |

System Menu tree description:

| Standard | Default set | Enter? | Restore factory default parameters |
| --- | --- | --- | --- |
|  | Address | 001-512 | Set light address code |
|  | DMX mode | Mode1 Mode2 Mode3 Mode4 | Select DMX control mode |
|  | Effect mode | No/Yes | effect disc automatically finds the shortest distance to rotate |
|  | Started lamp | Off/on | Choose whether the bulb is "on" or "off" every time you start it |
|  | Switch lamp | Off/on | Turn on the light bulb manually |
|  | No signal | Clear Hold | Keep DMX value or clear DMX channel value when there is no DMX signal |
|  | UI Color | Green Blue Golden Gray | Choose an on-screen menu color |
|  | Show time | No/Yes | Show runtime timer on main screen |
|  | Brightness | 000-255 | Adjust screen brightness |
|  | Screensaver | Off/on | Turn off the screen when there is no touch and key operation |
|  | XY encoder | No/Yes | Cancel or use XY automatic error correction function |
|  | X inversion | No/Yes | Select the X axis to run forward or backward |
|  | Y inversion | No/Yes | Select the Y axis to run forward or backward |
|  | Focus inversion | No/Yes | Select the focus axis to run forward or backward |
|  | Zoom inversion | No/Yes | Choose to run the magnification axis in forward or reverse direction |
| Advanced | This setting is the main parameter setting of the light, and permission is required |  |  |
| Info | Error List | No error/View | If there is no error or an error, click OK to view the error |
|  | System ver | Vxxxxxxx | System version information |
|  | Serial NO. | xxxxxxxx | Equipment factory number |
|  | SYS timer | 00000.0H | Total system running time (hours) |
|  | Run timer | 000:00 | Running time after this power-on (hours) |
|  | Lamp timer | 00000.0H | Total light bulb lighting time (hours)） |
|  | Equip TEMP | 000 | The temperature of the main parts of the device (requires device support) |
|  | Head TEMP | 000 | The temperature of the light head of the device (requires device support) |
| perform | Run mode | Auto/Sound | Run selected programs automatically or by voice |
|  | Run speed | 255 | Set the speed of the automatic running program |
|  | Run cross | 255 | Set slides to run automatically or with voice control (requires device support) |
|  | Built-in 1 | Off/on | Device Built-in Test Procedures 1 |
|  | Built-in 2 | Off/on | Device Built-in Test Procedures 2 |
|  | User PRO 1 | Off/on | User-programmed program 1 |
|  | User PRO 2 | Off/on | User-programmed program 2 |
|  | User PRO 3 | Off/on | User-programmed program 3 |
|  | User PRO 4 | Off/on | User-programmed program 4 |
|  | Circle shape | Off/on | (reserve) |
|  | Square shape | Off/on | (reserve) |
|  | Shape range | 000 | (reserve) |
|  | Sound DB | 000 | Adjust voice control sensitivity |
| Program |  |  |  |
| Reset | System reset to initial position |  |  |

24 DMX channel

| 24CH | Function | Value | Description |
| --- | --- | --- | --- |
| CH1 | Pan | 0-255 | 0-540° |
| CH2 | Pan fine | 0-255 | 0-2° |
| CH3 | Tilt | 0-255 | 0-270° |
| CH4 | Tilt fine | 0-255 | 0-1° |
| CH5 | Pan/Tilt Speed | 0-255 | Speed fast to slow |
| CH6 | Dimming | 0-255 | 0-100% dimming |
| CH7 | Strobe | 0-3 | off |
|  |  | 4-127 | Strobe from slow to fast |
|  |  | 128-131 | on |
|  |  | 132-251 | Random Strobe from slow to fast |
|  |  | 252-255 | on |
| CH8 | Color | 0-4 | White |
|  |  | 5-9 | Red 1 |
|  |  | 10-14 | Green 2 |
|  |  | 15-19 | Blue 3 |
|  |  | 20-24 | Yellow 4 |
|  |  | 25-29 | Orange red 5 |
|  |  | 30-34 | Blue green 6 |
|  |  | 35-39 | Rose 7 |
|  |  | 40-44 | Orange 8 |
|  |  | 45-49 | Light blue purple 9 |
|  |  | 50-54 | CTO 10 |
|  |  | 55-59 | CTB11 |
|  |  | 60-64 | White + Red |
|  |  | 65-69 | Red+ Green |
|  |  | 70-74 | Green + Blue |
|  |  | 75-79 | Blue + Yellow |
|  |  | 80-84 | Yellow + Rose |
|  |  | 85-89 | Rose + Blue green |
|  |  | 90-94 | Blue green+ Rose |
|  |  | 95-99 | Rose+ Orange |
|  |  | 100-104 | Orange+ Light blue purple |
|  |  | 105-109 | Light blue purple+ CTO |
|  |  | 110-114 | CTO+CTB |
|  |  | 115-119 | CTB+White |
|  |  | 120-185 | Forward flow from fast to slow |
|  |  | 186-189 | Stop |
|  |  | 190-255 | Reverse flow from slow to fast |
| CH9 | CMY1 | 0-255 | CMY Blue |
| CH10 | CMY2 | 0-255 | CMY Red |
| CH11 | CMY3 | 0-255 | CMY Yellow |
| CH12 | Fixed gobo | 0-5 | White |
|  |  | 6-11 | Gobo 1 |
|  |  | 12-17 | Gobo 2 |
|  |  | 18-23 | Gobo 3 |
|  |  | 24-29 | Gobo 4 |
|  |  | 30-35 | Gobo 5 |
|  |  | 36-41 | Gobo 6 |
|  |  | 42-47 | Gobo 7 |
|  |  | 48-53 | Gobo 8 |
|  |  | 54-59 | Gobo 9 |
|  |  | 60-65 | Gobo 10 |
|  |  | 66-71 | White 11 |
|  |  | 72-77 | Gobo 1 shake from slow to fast |
|  |  | 78-83 | Gobo 2 shake from slow to fast |
|  |  | 84-89 | Gobo 3 shake from slow to fast |
|  |  | 90-95 | Gobo 4 shake from slow to fast |
|  |  | 96-101 | Gobo 5 shake from slow to fast |
|  |  | 102-107 | Gobo 6 shake from slow to fast |
|  |  | 108-113 | Gobo 7 shake from slow to fast |
|  |  | 114-119 | Gobo 8 shake from slow to fast |
|  |  | 120-125 | Gobo 9 shake from slow to fast |
|  |  | 126-131 | Gobo 10 shake from slow to fast |
|  |  | 132-136 | Gobo 11 shake from slow to fast |
|  |  | 137-190 | Reverse flow from fast to slow |
|  |  | 191-192 | Stop |
|  |  | 193-255 | Forward flow from slow to fast |
| CH13 | Glass gobo | 0-6 | White |
|  |  | 7-13 | Gobo 1 |
|  |  | 14-20 | Gobo 2 |
|  |  | 21-27 | Gobo 3 |
|  |  | 28-34 | Gobo 4 |
|  |  | 35-41 | Gobo 5 |
|  |  | 42-48 | Gobo 6 |
|  |  | 49-55 | Gobo 7 |
|  |  | 56-62 | Gobo 8 |
|  |  | 63-69 | Gobo 1 shake from slow to fast |
|  |  | 70-76 | Gobo 2 shake from slow to fast |
|  |  | 77-83 | Gobo 3 shake from slow to fast |
|  |  | 84-90 | Gobo 4 shake from slow to fast |
|  |  | 91-97 | Gobo 5 shake from slow to fast |
|  |  | 98-104 | Gobo 6 shake from slow to fast |
|  |  | 105-111 | Gobo 7 shake from slow to fast |
|  |  | 112-118 | Gobo 8 shake from slow to fast |
|  |  | 119-184 | Forward flow from fast to slow |
|  |  | 185-255 | Reverse flow from slow to fast |
| CH14 | Gobo rotation | 0-127 | 0-360° |
|  |  | 128-190 | Reverse flow from fast to slow |
|  |  | 191-192 | Stop |
|  |  | 193-255 | Forward flow from slow to fast |
| CH15 | Prism 1 | 0-127 | no prism |
|  |  | 128-255 | prism 1 |
| CH16 | Prism 1 rotation | 0-127 | 0-400° |
|  |  | 128-187 | Forward flow from fast to slow |
|  |  | 188-195 | Stop |
|  |  | 196-255 | Reverse flow from slow to fast |
| CH17 | Prism 2 | 0-127 | no prism |
|  |  | 128-255 | prism 2 |
| CH18 | Prism 2 rotation | 0-127 | 0-400° |
|  |  | 128-187 | Forward flow from fast to slow |
|  |  | 188-195 | Stop |
|  |  | 196-255 | Reverse flow from slow to fast |
| CH19 | Frost | 0-127 | No effect |
|  |  | 128-255 | Frost |
| CH20 | Zoom | 0-255 | Small to large |
| CH21 | Focus | 0-255 | far to near |
| CH22 | Focus fine | 0-255 |  |
| CH23 | Auto | 0-49 | No effect |
|  |  | 50-99 | Pan auto |
|  |  | 100-149 | Tilt auto |
|  |  | 150-199 | Pan/Tilt auto |
|  |  | 200-255 | No effect |
| CH24 | Reset | 0-209 | No effect |
|  |  | 210-215 | Pan/Tilt reset after 6s |
|  |  | 220-235 | Effect motor reset after 6s |
|  |  | 240-255 | All reset after 6s |

Gobo wheel                          Color wheel

Glass gobo

CMY

DMX-512 connection

- Connect the provided XLR cable to the male 3-pin XLR output of your controller and the other side to the female 3-pin XLR input of the light.then connect the DMX signal to the first fixture to the 2nd fixture, after all fixtures are connected,

- The signal cable should be two - core with screened cable with XLR input and output connectors. Please refer to the diagram below

DMX terminator connection

If there are many lines or lights, it is recommended to use a DMX terminator to prevent DMX signal corruption, the DMX terminator is an simple XLR plug with a 120 Ω resistor connected between pins 2 and 3,which is then plugged into the output XLR socket of the last fixture in the chain.

3-Pins connection

Cleaning and maintenance

- Cleaning should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

Cut off power before cleaning and maintenance!

CAUTION

Attention

Do not see directly light source when open the light, and make sure power off before do any installation or maintenance.

Wires connection must be done professional person!

CAUTION

Cautions

Before delivery, this device has passed strict inspection, Please follow the user manual strictly for operation, if this fixture is damaged by improper operation and mistake, the fixture will be out of warranty, and manufacture or dealer won’t be responsible for it. In case of any technology change in this manual,we won’t advise in further .

Common issues instruction

| Issues | Solve instruction |
| --- | --- |
| No power | Check power’s voltage |
| When power in, output light but the device cannot accept control signal | Check address setting and the control DMX cable connection well or not |
| The effect have light shadow | Check electronic focus DMX channel value, if it’s suitable distance |
| Light beam dim | The lamp maybe arrived end of using life, should change new lamp, and also check light source cleaning or not |

Note:  the maintenance should be done by the verified technician.

Warranty Card This product is made of high-brightness transistor. We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product need maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：