- Display and buttons

Menu button：choose function

UP button: increase the parameters

DOWN button: decrease the parameters

ENTER button：press to confirm and save

- Menu function

Press MENU to choose the function, press DOWN and UP the choose the data, press ENTER to save (with memory function after turn off the light)

Menu details：

| A001 |  | A512 | press UP or DOWN to change DMX address（A001~A512）， press ENTER to save，default setting A001。 |
| --- | --- | --- | --- |
| CH03 |  | CH39 | press UP or DOWN to choose CH04, ch11, ch32 and ch39, press ENTER to save，default setting CH11。 |
| M000 |  | M083 | press UP or DOWN to choose 84 3in1 color macros (M000 ~ m083)，press ENTER to save，。 |
| S000 |  | S255 | press UP or DOWN to choose color marco’s speed(s000 ~ s255)，press ENTER to save. |
| M000 |  | M040 | press UP or DOWN to choose 41 white color marcos (M000 ~ M040) ,press ENTER to save. |
| S000 |  | S255 | press UP or DOWN to change white color marco’s speed(s000 ~ s255)，press ENTER to save。 |
| Soud |  | Soud | Sound activated mode |
| R255 |  | R000 | press UP or DOWN to change red LED brightness（R000~ R255）， press ENTER to save，default setting R255。 |
| G255 |  | G000 | press UP or DOWN to change green LED brightness（G000~ G255）, press ENTER to save，default setting G255。 |
| B255 |  | B000 | press UP or DOWN to change blue LED brightness（B000~ B255），press ENTER to save，default setting B255。 |
| W255 |  | B000 | press UP or DOWN to change white LED brightness（W000~ W255), press ENTER to save，default setting W255。 |
| T000 |  |  | Inner temperature，T045 means current fixture temperature is 45℃；if not install 10K thermistor，it displays T000。 |

- Master/slave control

Two or more of the same lights are connected with DMX three-core signal line, the light is set to any address code of A001~A512, any one is set as the master, and the other lamps are the slaves, and all the slaves display screens do not flash; use the master to fade , Pulse change, jump change, sound control, and auto effects, all slaves will synchronize gradual change, pulse change, jump, sound control, and auto effects.

Special attention:

1. Only one host can be set for a group of lights. If there are more than one host, all lamps will flash randomly and out of sync.

2. All lights can be the master/slave mode when the DMX512 console is turned off.

4、Factory setting

At menu DMX address setting A001-A512, press and hold MENU for 3 seconds to enter factory setting.  Factory settings are mainly the functions of each output power of the lamp, fan setting mode, setting temperature protection point, and sending parameters. Any mode in the factory setting can be exited by pressing the menu button for 3 seconds.

Factory setting table:

| R255 |  | R032 | press UP or DOWN to change red LED brightness（R032-R255）， press ENTER to save，default setting R200。 |
| --- | --- | --- | --- |
| G255 |  | G032 | press UP or DOWN to change green LED brightness (R032-R255）， press ENTER to save，default setting R200。 |
| B255 |  | B032 | press UP or DOWN to change blue LED brightness (R032-R255）， press ENTER to save，default setting R200。 |
| W255 |  | W032 | press UP or DOWN to change white LED brightness（R032-R255）， press ENTER to save，default setting R200。 |
| FAN0 |  | FAN1 | Fan setting：FAN0 means fan starts working when LED gets ON, FAN1 means when the inside temperature reach a certain temperature protection point the fan starts working， press ENTER to save。 |
| T040 |  | T070 | Set temperature protection point for fan，press UP or DOWN to choose（40℃~70℃），press ENTER to save，default setting T065。 |
| Send |  | Send | Send the fixture default settings to other fixtures which are connected with 3pin DMX cables, press MENU to send default settings, press ENTER to cancel sending default settings. |

5、DMX512control

Set the DMX address for fixture in menu,l then connect the fixtures with 3pin XLR signal cable with DMX controller, the DMX signal indicator stops flashing, it means the DMX controller sends the signal to fixture successfully, and can control the light with DMX controller.

CH04 DMX channel：

| channel | channel data | function |
| --- | --- | --- |
| 1 | 000-255 | red led dimming 0-100% |
| 2 | 000-255 | green led dimming 0-100% |
| 3 | 000-255 | blue led dimming 0-100% |
| 4 | 000-255 | white led dimming 0-100% |

CH11 DMX channel：

| channel | channel data | function |
| --- | --- | --- |
| 1 | 000-255 | master dimmer |
| 2 | 000-255 | RGB strobe |
| 3 | 000-255 | RGB mode（see mode effect 1） |
| 4 | 000-255 | RGB mode speed |
| 5 | 000-255 | red led dimming 0-100% |
| 6 | 000-255 | green led dimming 0-100% |
| 7 | 000-255 | blue led dimming 0-100% |
| 8 | 000-255 | white strobe |
| 9 | 000-255 | White mode（see mode effect 2） |
| 10 | 000-255 | White mode speed |
| 11 | 000-255 | white led dimming 0-100% |

CH32 DMX channel：

| channel | channel data | function |
| --- | --- | --- |
| 1 | 000-255 | R section 1 red led dimming 0-100% |
| 2 | 000-255 | G section 1 green led dimming 0-100% |
| 3 | 000-255 | B section 1 blue led dimming 0-100% |
| 4 | 000-255 | R section 2 red led dimming 0-100% |
| 5 | 000-255 | G section 2 green led dimming 0-100% |
| 6 | 000-255 | B section 2 blue led dimming 0-100% |
| … | … | … … |
| 22 | 000-255 | R section 8 red led dimming 0-100% |
| 23 | 000-255 | G section 8 green led dimming 0-100% |
| 24 | 000-255 | B section 8 blue led dimming 0-100% |
| 25 | 000-255 | section 1 white led dimming 0-100% |
| 26 | 000-255 | section 2 white led dimming 0-100% |
| 27 | 000-255 | section 3 white led dimming 0-100% |
| 28 | 000-255 | section 4 white led dimming 0-100% |
| 29 | 000-255 | section 5 white led dimming 0-100% |
| 30 | 000-255 | section 6 white led dimming 0-100% |
| 31 | 000-255 | section 7 white led dimming 0-100% |
| 32 | 000-255 | section 8 white led dimming 0-100% |

CH39 DMX channel：

| channel | channel data | function |
| --- | --- | --- |
| 1 | 000-255 | master dimmer |
| 2 | 000-255 | RGB strobe |
| 3 | 000-255 | RGB mode（see mode effect 1） |
| 4 | 000-255 | RGB mode speed |
| 5 | 000-255 | section 1 red led dimming 0-100% |
| 6 | 000-255 | section 1 green led dimming 0-100% |
| 7 | 000-255 | section 1 blue led dimming 0-100% |
| … | … | … … |
| 26 | 000-255 | section 8 red led dimming 0-100% |
| 27 | 000-255 | section 8 green led dimming 0-100% |
| 28 | 000-255 | section 8 blue led dimming 0-100% |
| 29 | 000-255 | white strobe |
| 30 | 000-255 | White mode（see mode effect 2） |
| 31 | 000-255 | white mode speed |
| 32 | 000-255 | section 1 white led dimming 0-100% |
| 33 | 000-255 | section 2 white led dimming 0-100% |
| 34 | 000-255 | section 3 white led dimming 0-100% |
| 35 | 000-255 | section 4 white led dimming 0-100% |
| 36 | 000-255 | section 5 white led dimming 0-100% |
| 37 | 000-255 | section 6 white led dimming 0-100% |
| 38 | 000-255 | section 7 white led dimming 0-100% |
| 39 | 000-255 | section 8 white led dimming 0-100% |

6、mode effect

1.RGB mode effect:

| Value | Mode no. | effect |
| --- | --- | --- |
| 0-2 | 0 | open |
| 3-5 | 1 | R |
| 6-8 | 2 | G |
| 9-11 | 3 | B |
| 12-14 | 4 | RG |
| 15-17 | 5 | RB |
| 18-20 | 6 | GB |
| 21-23 | 7 | RGB |
| 24-26 | 8 | cycle mode 1-7 |
| 27-29 | 9 | color fading |
| 30-32 | 10 | pulse strobe |
| 33-35 | 11 | 1 segment R racing |
| 36-38 | 12 | 1 segment G racing |
| 39-41 | 13 | 1 segment B racing |
| 42-44 | 14 | 1 segment RG racing |
| 45-47 | 15 | 1 segment RB racing |
| 48-50 | 16 | 1 segment GB racing |
| 51-53 | 17 | 1 segment RGB racing |
| 54-56 | 18 | cycle mode 11-17 |
| 57-59 | 19 | 2 segments R racing |
| 60-62 | 20 | 2 segments G racing |
| 63-65 | 21 | 2 segments B racing |
| 66-68 | 22 | 2 segments RG racing |
| 69-71 | 23 | 2 segments RB racing |
| 72-74 | 24 | 2 segments GB racing |
| 75-77 | 25 | 2 segments RGB racing |
| 78-80 | 26 | cycle mode 19-25 |
| 81-83 | 27 | 1 segment R refresh |
| 84-86 | 28 | 1 segment G refresh |
| 87-89 | 29 | 1 segment B refresh |
| 90-92 | 30 | 1 segment RG refresh |
| 93-95 | 31 | 1 segment RB refresh |
| 96-98 | 32 | 1 segment GB refresh |
| 99-101 | 33 | 1 segment RGB refresh |
| 102-104 | 34 | cycle mode 27-33 |
| 105-107 | 35 | red LED in front and rear segment refresh back and forth。 |
| 108-110 | 36 | green LED in front and rear segment refresh back and forth。 |
| 111-113 | 37 | blue LED in front and rear segment refresh back and forth。 |
| 114-116 | 38 | R+G LED in front and rear refresh back and forth。 |
| 117-119 | 39 | R+B LED in front and rear refresh back and forth。 |
| 120-122 | 40 | G+B LED in front and rear refresh back and forth。 |
| 123-125 | 41 | RGB LED in front and rear refresh back and forth。 |
| 126-128 | 42 | cycle mode 35-41 |
| 129-131 | 43 | 2 segments R racing back and forth |
| 132-134 | 44 | 2 segments G racing back and forth |
| 135-137 | 45 | 2 segments B racing back and forth |
| 138-140 | 46 | 2 segments RG racing back and forth |
| 141-143 | 47 | 2 segments RB racing back and forth |
| 144-146 | 48 | 2 segments GB racing back and forth |
| 147-149 | 49 | 2 segments RGB racing back and forth |
| 150-152 | 50 | cycle mode 43-49。 |
| 153-155 | 51 | R+G racing back and forth |
| 156-158 | 52 | G+B racing back and forth |
| 159-161 | 53 | B+RG racing back and forth |
| 162-164 | 54 | G+RB racing back and forth |
| 165-167 | 55 | R+BG racing back and forth |
| 168-170 | 56 | GB+GB racing back and forth |
| 171-173 | 57 | RGB+R racing back and forth |
| 174-176 | 58 | cycle mode 51-57 |
| 177-179 | 59 | R of 2segments racing |
| 180-182 | 60 | G of 2segments racing |
| 183-185 | 61 | B of 2segments racing |
| 186-188 | 62 | RG of 2segments racing |
| 189-191 | 63 | RB of 2segments racing |
| 192-194 | 64 | BG of 2segments racing |
| 195-197 | 65 | RGB of 2segments racing |
| 198-200 | 66 | cycle mode 59-65 |
| 201-203 | 67 | R racing with tail |
| 204-206 | 68 | G racing with tail |
| 207-209 | 69 | B racing with tail |
| 210-212 | 70 | RG racing with tail |
| 213-215 | 71 | RB racing with tail |
| 216-218 | 72 | GB racing with tail |
| 219-221 | 73 | RGB racing with tail |
| 222-224 | 74 | cycle mode 67-73 |
| 225-227 | 75 | 1 segment red stacking |
| 228-230 | 76 | 1 segment green stacking |
| 231-233 | 77 | 1 segment blue stacking |
| 234-236 | 78 | 1 segment R+G stacking |
| 237-239 | 79 | 1 segment R+B stacking |
| 240-242 | 80 | 1 segment B+G stacking |
| 243-245 | 81 | 1 segment RGB stacking |
| 246-248 | 82 | cycle mode 75-81。 |
| 249-251 | 87 | color stacking |
| 252-254 | 88 | color flowing |
| 255 | 89 | cycle mode 11 ~ 81，can dim RGB DMX channel to change background color |

2.white mode effect:

| Value | Mode no. | effect |
| --- | --- | --- |
| 0-5 | 1 | open |
| 6-11 | 2 | segment 1 white |
| 12-17 | 3 | segment 2 white |
| 18-23 | 4 | segment 3 white |
| 24-29 | 5 | segment 4 white |
| 30-35 | 6 | segment 5 white |
| 36-41 | 7 | segment 6 white |
| 42-47 | 8 | segment 7 white |
| 48-53 | 9 | segment 8 white |
| 54-59 | 19 | 1 segment white running from left to right |
| 60-65 | 20 | 1 segment white running from right to left |
| 66-71 | 23 | 2 segments white running from left to right |
| 72-77 | 24 | 2 segments white running from right to left |
| 78-83 | 27 | 3 segments white running from left to right |
| 84-89 | 28 | 3 segments white running from right to left |
| 90-95 | 30 | 1 segment white running back and forth |
| 96-101 | 33 | 2 segments white running back and forth |
| 102-107 | 34 | 3 segments white running back and forth |
| 108-113 | 37 | white running from left to right with tail |
| 114-119 | 39 | white running from right to left with tail |
| 120-125 | 40 | white running from left to right with tail |
| 126-131 | 43 | white running from right to left with tail |
| 132-137 | 45 | White running back and forth with tail |
| 138-143 | 47 | 1 segment white LED refresh from left to right |
| 144-149 | 49 | 1 segment white LED refresh from right to left |
| 150-155 | 51 | front and rear segment refresh from side to middle |
| 156-161 | 53 | refresh from middle to both sides |
| 162-167 | 55 | 2 side white running back and forth |
| 168-173 | 57 | 1 segment white stacking from left to right |
| 174-179 | 59 | 1 segment white stacking from right to left |
| 180-185 | 61 | white wave racing from left to right |
| 186-191 | 63 | white wave racing from right to left |
| 192-197 | 65 | 1 white racing from 2 sides to middle |
| 198-203 | 67 | 1 white racing from middle to 2 sides |
| 204-209 | 69 | 1 white fresh back and forth in 5 rounds |
| 210-215 | 71 | white of 4 connecting segments racing back and forth |
| 216-221 | 73 | 1 segment move from left to right |
| 222-227 | 75 | 1 segment move from right to left |
| 228-233 | 77 | 1 color fading white running back and forth |
| 234-239 | 79 | 2 white shaking |
| 240-245 | 81 | white pulse strobe |
| 246-251 | 87 | white's 2 ends meets and become bigger |
| 252-255 | 88 | cycle modes |

- Technical data

Voltage：AC100 ~240V  50/60Hz，

Power consumption: 185W

LED: 960pcs SMD5050 RGB LED

Control mode ：Master-Slave/Auto/DMX512/Sound/ RDM

DMX channel：4CH/11CH/32CH/39CH

Dimming: 32bit, 0~100% linear dimming

Features: 8-segment horse chasing + wash + strobe

Working Temp.:-30°~ 50°

Strobe: 1~30-Hz；

Case: metal, black

Connection: DMX input/output,Power input/output

IP rating: IP20