2000mW RGB Animation Mini Laser

User Manual

Please read this manual carefully before use!

Contents

Introduction--------------------------------------------------------------------------------------------P2

Silkscreen on product------------------------------------------------------------------------------P3

Safety instruction/Installation/Technical specification---------------------------------P4

MENU----------------------------------------------------------------------------------------------------P5

DMX512 Channel----------------------------------------------------------------------------------P5-7

DMX512 connection-------------------------------------------------------------------------------P7

DMX address setting/Ringing-----------------------------------------------------------------P7-8

Cleaning and maintenance/Warning------------------------------------------------------- P8

Warranty card---------------------------------------------------------------------------------------P9

- Introduction

Thank you for purchasing this product. To optimize the performance of this product,prior to use,please read these operating instructions carefully to familiarize yourself with the basic operation of this unit.Please keep the manual in safe price for future reference.

This unit has been tested at our factory before being shipped to you, there is no assembly required.

Once after received this product , please take and put carefully. And check cautiously that whether the product was damaged or not during the transportation.

- Checking Parts

Please check if the following parts are included:

1 x Laser light

1 x User manual

1 x Power cable

Caution

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Symbol Instruction On The Device

…

-------------------------------------------------------------------------------CE Certificate

---------------------------------------------------------------------Only indoor use

------------------------------------------------------------------------Environmental protection

----------------------------------------The minimum distance from lighted objects(meter)

------------------------------------------------------------Max. Ambient Temp

----------------------------------------------------Max. Housing Temp

------------------------------------------------------------------------- Safety caution

-------------------------------------------------------------------------Careful burns

------------------------------------------------------------------------Careful electric shock

-----Not suitable for directly installed on the surface of ordinary combustible materials

Safety Instructions

- Please open and check whether break or not for transport once receive this fixture, if yes, not use it and contact dealer or manufacture as soon as possible.

- The Max applicable temperature for the light is 40°C. Do not use the light under or above the temperature. ta=40℃

- The fixture is applicable for many occasions,  it should keep clean, and avoid long use in overheating or dusty environment. To prevent the fixture from the chemical liquid .

- User must be careful of fire, heat, electric and fall-off that make heavy or fatal injury. Please read this user manual carefully before power on and installation, following safety and operation instruction in it. If still have any questions, please contact dealer or manufacture.

- User must following this user manual, do not open fixture by yourself, if any problems, please contact professional technician, maintenance should by technician.

Do not power on or start the fixture before installation, please stop after working more than 8 hours, which will longer use life.

- Fixed installation, do not by strong shocks.

- The minimum distance between light and the illuminated surface must be more than 1 meter.

- Never look directly into the light source to avoid hurt to eyes (especially epileptics)!

When dustproof glass at light output breaks, cracks or other visible damage, user should replace with  new glass to continue to use.

Installation

- When install this equipment please make sure there's no flammable surfaces (decorated things, etc) and minimum distance of 0.5M from the equipment to the flammable.

- The installation of the effect has to be built and constructed in a way that it can hold 10 times the weight and without any harming deformation.

- The installation must always be secured with a secondary safety attachment, e.g. an appropriate safety rope.Never stand directly below the device when mounting, removing or servicing the fixture.

Technical Specifications

Input voltage: AC100-240V, 50/60Hz

Fuse: BGDP ￠5x20mm   250VAC  F2A

Total power:20W

Light source: 2000mW(R:400mW/630nm+G:500mW/520nm+B:1200mW/450nm)

Control mode: Auto/Sound/DMX512/Master-slave

Channels：16CH

Scanning system: 15Kpps Scanner, ±20º

51 internal patterns for show

Surface temp: Tb=45ºC ( this data testing when the environment temp is 30ºC)

Working temp: 10ºC--65ºC

Modulation: TTL

N.W: 2.39Kg

Product size: 188*193*173mm

Laser control system

1. MENU display

| MENU | Display | Function |
| --- | --- | --- |
| Addr | 1-512 | DMX address setting |
| AUTO | 0-9 | Auto mode |
| SOUN | 0-9 | Sound mode |
| VERSION | Ver1.1 | VERSION |

DMX Channel Instructions

| Channel | Function | Value | Details |
| --- | --- | --- | --- |
| CH1 | Laser On/Off | 0-9 | Laser off |
|  |  | 10-99 | Laser on |
| CH2 | Color control | 0-69 | White-Red-Blue-Pink-Cyan-Yellow-Green（Fixed color option） |
|  |  | 70-79 | Whole color change (Eotic speed controlled by CH3) |
|  |  | 80-89 | Default color (Eotic speed controlled by CH3) |
|  |  | 90-99 | 7-colors (Eotic speed controlled by CH3) |
|  |  | 100-224 | Segment color, 5 data as one different (Eotic speed controlled by CH3) |
|  |  | 225-229 | Dynmtic eotic color 1 (Eotic speed controlled by CH3) |
|  |  | 230-234 | Dynmtic eotic color 2 (Eotic speed controlled by CH3) |
|  |  | 235-239 | Dynmtic eotic color 3 (Eotic speed controlled by CH3) |
|  |  | 240-244 | Dynmtic eotic color 4 (Eotic speed controlled by CH3) |
|  |  | 245-249 | Dynmtic eotic color 5 (Eotic speed controlled by CH3) |
|  |  | 250-255 | Dynmtic eotic color 6 (Eotic speed controlled by CH3) |
| CH3 | Color Eotic speed | 0-9 | No function |
|  |  | 10-127 | Color clockwise eotic, speed from slow to fast |
|  |  | 128-255 | Color anti-clockwise eotic, speed from slow to fast |
| CH4 | Pattern option | 0-255 | Pattern option (patterns segment by CH5, total 5 groups) |
| CH5 | Patterns group option | 0-50 | Inner patterns group 1 |
|  |  | 51-101 | Inner patterns group 2 |
|  |  | 102-152 | Inner patterns group 3 |
|  |  | 153-203 | Inner patterns group 4 |
|  |  | 204-255 | Inner patterns group 5 |
| CH6 | Pattern size | 0-255 | Pattern size manual option |
| CH7 | Pattern auto zoom | 0-15 | Pattern manual zoom |
|  |  | 16-55 | Zoom from small to big, speed from slow to fast |
|  |  | 56-95 | Zoom from big to small, speed from slow to fast |
|  |  | 96-135 | Cycle zoom big to small, speed from slow to fast |
|  |  | 136-175 | Anomaly cycle zoom 1，speed from slow to fast |
|  |  | 176-215 | Anomaly cycle zoom 2，speed from slow to fast |
|  |  | 216-255 | Anomaly cycle zoom 3，speed from slow to fast |
| CH8 | center rotation | 0-127 | Rotation angle option |
|  |  | 128-191 | Clockwise rotation speed option |
|  |  | 192-255 | Anti-Clockwise rotation speed option |
| CH9 | Horizontal rotation | 0-127 | Flip horizontal location option |
|  |  | 128-255 | Flip horzontal speed option |
| CH10 | Vertical rotation | 0-127 | Flip vertical location option |
|  |  | 128-255 | Flipvertical speed option |
| CH11 | Horizontal Remove | 0-127 | Horizontal location option |
|  |  | 128-255 | Horizontal auto location, speed from slow to fast |
| CH12 | Vertical Remove | 0-127 | Vertical rotation |
|  |  | 128-255 | Vertical auto location, speed from slow to fast |
| CH13 | Wave | 0-9 | No function |
|  |  | 10-255 | Wave range and speed（Range from small to big, speed from Slow to fast, every 32 data as one group, total 8 group) |
| CH14 | Patterns drawing | 0-1 | No function |
|  |  | 2-63 | Manual drawing 1 |
|  |  | 64-127 | Manual drawing 2 |
|  |  | 128-153 | Auto drawing（increase） |
|  |  | 154-179 | Auto drawing（decrease） |
|  |  | 180-205 | Auto drawing（from increase to decrease-reverse） |
|  |  | 206-255 | Auto drawing（from increase to decrease-syntropy） |
| CH15 | Inner dynamic effect | 0-2 | No function |
|  |  | 3-229 | Single group dynamic effect, speed and color controlled by CH16 |
|  |  | 230-249 | Random auto effect, speed and color controlled by CH16 |
|  |  | 250-255 | Sound |
| CH16 | Inner dynamic effect speed | 0-127 | Inner effect speed from fast to slow, color fixed by inner programme |
|  |  | 128-255 | Inner effect speed from fast to slow, color fixed by DMX |

- DMX-512 connection

t light (Address 001)         Second light (Address 017)         Third light(Address 033)

DMX Address setting

Each fixture must set a specific starting address. When receiving signal transmission, fixture will receive channel control signals which from the starting address.

According to different requirements or easy to use, the user can set many value fixtures to the same address, also can set up a separate address for every device.

If you set the same address,all the devices will receive from DMX channels signal. All connecting fixtures are jointly controlled, controller can't separate control a device.

If you set a different address, each device will be separate from its particular set the starting address and begin accepting DMX512 control signal, so that more convenient separate control one device. Using this method must be based on channel quantity of device to determine a starting address.

This laser light has 16 channels.you should set the starting address of the first unit to 1, the second unit to 17(1 + 16), the third unit to 33 (16*2+ 1), and so on.

Ringing

1 Mounting bracket

2 Handle

- .The ambient temperature range is 1°C--45°C, must never be exceeded.

- .The installation of the projector has to be built and constructed in a way that it can hold 10 times the weight for 1 hour without any harming deformation.

- ).When rigging or servicing the fixture staying in the area below the installation place, on  bridges,under high working places and other endangered areas is forbidden.

D).The operator has to make sure that safety-relating  and machine-technical installations are approved by an expert before taking into operation for the first time and after changes before taking into operation another time.

E).The operator  has to make sure that safety-relating  and machine-technical installations  are  approved by a skilled person once a year.

F). Sure M8 screws on mounting bracket fixed middle hole ¢12.5mm of handle

Cleaning and maintenance

- Maintenance should be performed every 15-day period, by using a sponge which is dipped with alcohol, rather than wet cloth or other chemical liquid, to clean the mirror.

- Always disconnect from the mains when the device is not in use or before cleaning it.

Caution

Make sure power off before cleaning and maintenance!

Warning

Warning!

Never look directly into the light source,as sensitive persons may suffer an epileptic shock(especially meant for epileptics)!

Caution

Warning!

Any power connected must by professional person

Warranty Card This product is made of high-brightness transistor. We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product need maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：