## FAT BEAM LASER BAR

- Please read this manual carefully before use!

Contents

Introduction And Caution------------------------------------------------------------P2

Symbol Instruction And Safety Instruction--------------------------------------P3

Installation Instruction----------------------------------------------------------------P4

Product Specification And Manu Operation ------------------------------------P5~P6

DMX Chart ------------------------------------------------------------------------------P7~P18

DMX Connection ----------------------------------------------------------------------P18~P19

Troubleshooting And Fixture Cleaning--------------------------------------------P19-P20

Warranty card---------------------------------------------------------------------------P21

Thank you for your patronage. We are confident that our excellent products and service can satisfy you. For your own safety, please read this user manual carefully before installing the device.

In order to install , operate, and maintain the light correctly, we suggest that the installation and operation should be done by the verified technician with following the instruction strictly.

This moving laser bar’s output is amazing. It is well designed with professional aluminum alloy housing. It makes full use of the advanced electronic positioning technology, and with international standard DMX512 control system. It is ideal for TV show, disco, dancing hall, night club, stage...and some applications outdoor.

Parts inside the box:

| Item | Quantity | Unit |
| --- | --- | --- |
| Light fixture | 1 | pc |
| User Manuel | 1 | pc |
| Power Cable | 1 | pc |
| DMX connection cable | 1 | pc |
| Safety Rope | 1 | pc |
| Handle | 2 | pcs |

Please check carefully that there is no damage caused by transportation. Please contact your local dealer if any problem.

Caution

Before leaving the factory, this unit has passed strict inspection, functions stable and perfect packing. If this unit is operated in any way different to the one described in this manual, the product may suffer damages and the guarantee becomes void. Furthermore, any other operation may lead to dangers like short circuit, burns, electric shock, crash etc. If any technology changes of this manual, company do not notice in further and have interpretation.

CAUTION!

Keep this device away from rain and moisture!

Unplug mains lead before opening the housing!

Warning!

Damages caused by the disregard of this user manual are not subject to warranty. The dealer will not accept liability for any resulting defects or problems.

Symbol Instruction On The Device

……………………………………………………………

…

CE Certificate

………

…

……………………………………………………

…

……………………………………………

…

…………………

…

Suitable directly installed on the common combustible materials devices

Only indoor using

Environmental protection

The minimum distance from lighted objects(meter)

Safety Instructions

This unit was in perfect condition before leaving factory. In order to maintain this condition and to ensure a safe operation, it is absolutely necessary for the user to follow the safety instructions and warning notes written in this user manual.

- ►Unpack and check carefully there is no transportation damage before using the unit.

- ►Disconnect main power before replacement or servicing.

- ►The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent

- surfaces. Be sure that no ventilation slots are blocked.

- ►Maximum ambient temperature is Ta: 40 degrees C. DO NOT operate it where the temperature is

- higher than this.

- ►In the event of a serious operating problem, stop using the unit immediately. Never try to repair the

- unit by yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please

- contact the nearest authorized technical assistance center. Always use the same type spare parts.

- ►DO NOT connect any dimmer.

- ►DO NOT touch any cables during operation as high voltage may cause electric shock.

- ►To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or moisture.

- ►After unit working for 8 hours continuously, let the unit to take appropriate rest. It can reduce some damage and good for lifetime.

- ►Laser radiation avoids body exposure, especially for eye, it’s easy to cause blindness. Please keep the

- safe distance at 1 meters above.

- ►To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or moisture.

- ►The housing and lenses must be replaced if they are visibly damaged.

CAUTION!

Please consider the GB7000.15/EN60598-2-17 and the other respective national norms during

The installation. The installation must only be carried out by a qualified person.

Installation Instruction

- This unit can not be installed on the surface of flammable materials. Please ensure the unit is at least 0.5m away from flammable and explosive materials.

- The installation must always be secured with a secondary safety e.g. an appropriate safety rope.Never stand directly below the device when mounting, removing or servicing the fixture.

- The applicable temperature for the lighting is between -10°C to 45°C. Do not use the lighting under or above the temperature.

- Before installation, must ensure installation bracket, handle or truss for this light can hold 10 times of the fixture weight for 1 hour without any harming deformation.

- The operator has to make sure the safety relating and machine technical installations are approved by an expert before taking the device into operation for the first time.

- These installations have to be approved by a skilled person once a year.

- Overhead mounting requires extensive experience, including others calculating working load limits, installation material being used, and periodic safety inspection of all installation material and the device. If you lack these qualifications, do not attempt the installation yourself. Improper installation can result in bodily injury.

Insert the quick-lock fasteners of the first holder into the respective holes on the bottom of the device.Tighten the quick-lock fasteners fully clockwise.

Make the finished installed mounting bracket tighten on the fixed bracket.

Pull the safety rope through the holes on the bottom of the base and over the trussing system or other safe fixation spot.

Note:This step is quite important to ensure that fixture will not drop out by the damage of clamp.

Product specification

Red laser diode: 100mW/638nm+50mW/515nm+150mW/450nm ; average lifetime is over 2000 hours, low power consumption.

DMX channel：9CH/19-1CH/19-2CH/32CH

Control mode ：Master/Slave, Auto, DMX512, Sound

Pan: 240º, Tilt: 90º

Electrical strobe: 1~23Hz；

Input voltage：AC100 ~240V  50/60Hz，

Power consumption: 120W

3pin DMX input/output, Power Con input/output

Ambient. Temp:-10°-- 40°

Display

MENU Nap

| Nenu （MENU） | Addr (Address setting) | enter | +/- | set address（001-512） | enter(save and exit) |
| --- | --- | --- | --- | --- | --- |
|  | Chnd (Channel option) | enter | +/- | 9 Ch | enter(save and exit) |
|  |  |  |  | 19_1Ch | enter(save and exit) |
|  |  |  |  | 19_2Ch | enter(save and exit) |
|  | SLnd (Master/Slave) | enter | +/- | Mast | enter(save and exit) |
|  |  |  |  | SL 1 | enter(save and exit) |
|  | Lost (Without DMX) | enter | +/- | blac (Master) | enter(save and exit) |
|  |  |  |  | hold | enter(save and exit) |
|  |  |  |  | auto (Auto) | enter(save and exit) |
|  |  |  |  | soun (Sound) | enter(save and exit) |
|  | Sens (Sound sensitivity) | enter | +/- | SE00-SE99 | enter(save and exit) ） |
|  | Sped (Auto Speed) | enter | +/- | SP00-SP99 | enter(save and exit) |
|  | Inur (Reverse setting) | enter | +/- | Pan --> YES/NO | enter(save and exit) |
|  |  |  |  | Til1 --> YES/NO | enter(save and exit) |
|  |  |  |  | Til2 --> YES/NO | enter(save and exit) |
|  |  |  |  | Til3 --> YES/NO | enter(save and exit) |
|  |  |  |  | Til4 --> YES/NO | enter(save and exit) |
|  |  |  |  | Til5 --> YES/NO | enter(save and exit) ） |
|  |  |  |  | Til6 --> YES/NO | enter(save and exit) |
|  | Nadj (Motor adjustment) | enter | +/- | Padj --> 000-255 | enter(save and exit) |
|  |  |  |  | T1aj --> 000-255 | enter(save and exit) |
|  |  |  |  | T2aj --> 000-255 | enter(save and exit) |
|  |  |  |  | T3aj --> 000-255 | enter(save and exit)） |
|  |  |  |  | T4aj --> 000-255 | enter(save and exit) |
|  |  |  |  | T5aj --> 000-255 | enter(save and exit) |
|  |  |  |  | T6aj --> 000-255 | enter(save and exit) |
|  | Test (Programme test) | enter | +/- | Function test | enter(save and exit) |
|  | Led (Black-out) | enter | +/- | on | enter(save and exit) |
|  |  |  |  | off | enter(save and exit) |
|  | disp (Display reverse) | enter | +/- | on | enter(save and exit) |
|  |  |  |  | off | enter(save and exit) |
|  | Fhrs (Working time) | enter |  | Working time | enter(save and exit) |
|  | Ver (Software version) | enter |  | Software version | enter(save and exit) |
|  | REST | enter |  | Rest function | enter(save and exit) |
|  | defa (Factory default) | enter |  | Factory default | enter(save and exit) |

MENU operation

This fixture has 4 keys on panel for operation, press these keys to set DMX address, reset, edit and run programs and so on.

[MENU]: press MENU key to enter edit status, in edit status, if press MENU again, it will return the upper interface. If no operation 30 seconds after enter edit status, the fixture will exit the edit status automatically.

[Up]: press UP key, the indicator will flash, the address increases one by one, if hold the key continuously, the address will increase rapidly. If no press within 30 seconds, the indicator stops flashing and exit it.  In edit status, press UP key, it will display the options circularly from down to up.

[Down]：press DOWN key, the indicator will flash, the address decreases one by one, if hold the key continuously, the address will decrease rapidly. If no press within 30 seconds, the indicator stops flashing and exit it.  In edit status, press DOWN key, it will display the options circularly from up to down.

[ENTER]：in normal status, no function after press ENTER. In edit mode, press ENTER to enter lower interface.

DMX512 Channel

- 9 DMX Channel chart

| Channel |  | Value | Function |
| --- | --- | --- | --- |
| CH1 | Pan | 000-255 | Pan angle：0-260° |
| CH2 | Pan Speed | 000-255 | Pan speed from slow to fast |
| CH3 | Tilt | 000-255 | Tilt 1-6；Pan：0-90° |
| CH4 | Tilt Speed | 000-255 | Tilt speed from slow to fast |
| CH5 | Dimmer | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH6 | Strobe | 000-015 | OFF |
|  |  | 016-030 | ON |
|  |  | 031-090 | Variable strobe from slow to fast |
|  |  | 091-105 | ON |
|  |  | 106-165 | Pulse effect from slow to fast |
|  |  | 166-180 | ON |
|  |  | 181-240 | Random strobe from slow to fast |
|  |  | 241-255 | ON |
| CH7 | Laser Macro1-6 | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH8 | Reset / Auto | 000-008 | NO FUNCTION |
|  |  | 009-019 | SHOW1 |
|  |  | 020-030 | SHOW2 |
|  |  | 031-041 | SHOW3 |
|  |  | 042-052 | SHOW4 |
|  |  | 053-063 | SHOW5 |
|  |  | 064-074 | SHOW6 |
|  |  | 075-085 | SHOW7 |
|  |  | 086-096 | SHOW8 |
|  |  | 097-107 | SHOW9 |
|  |  | 108-118 | SHOW10 |
|  |  | 119-129 | SHOW11 |
|  |  | 130-140 | SHOW12 |
|  |  | 141-151 | SHOW13 |
|  |  | 152-162 | SHOW14 |
|  |  | 163-173 | SHOW15 |
|  |  | 174-184 | SHOW16 |
|  |  | 185-195 | SHOW17 |
|  |  | 196-206 | SHOW18 |
|  |  | 207-217 | SHOW19 |
|  |  | 218-228 | SHOW20 |
|  |  | 229-239 | SHOW21 |
|  |  | 240-250 | Auto effect |
|  |  | 251-255 | Reset |
| CH9 | Auto program / Sound mode | 000-250 | Program speed from slow to fast |
|  |  | 251-255 | Sound mode |

- 19-1 DMX Channel chart

| Channel |  | Value | Function |
| --- | --- | --- | --- |
| CH1 | Pan | 000-255 | Pan angle：0-260° |
| CH2 | PanSpeed | 000-255 | Pan speed from fast to slow |
| CH3 | Tilt1 | 000-255 | Tilt angle：0-90° |
| CH4 | Tilt2 | 000-255 | Tilt angle：0-90° |
| CH5 | Tilt3 | 000-255 | Tilt angle：0-90° |
| CH6 | Tilt4 | 000-255 | Tilt angle：0-90° |
| CH7 | Tilt5 | 000-015 | Tilt angle：0-90° |
| CH8 | Tilt6 | 000-255 | Tilt angle：0-90° |
| CH9 | TiltSpeed | 000-255 | Pan speed from fast to slow |
| CH10 | Dimmer | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH11 | Strobe | 000-015 | OFF |
|  |  | 016-030 | ON |
|  |  | 031-090 | VARIABLE STROBE SLOW TO FAST |
|  |  | 091-105 | ON |
|  |  | 106-165 | PULSE EFFECT SLOW TO FAST |
|  |  | 166-180 | ON |
|  |  | 181-240 | RANDOM STROBE SLOW TO FAST |
|  |  | 241-255 | ON |
| CH12 | Laser 1 Macro | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH13 | Laser 2 Macro | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH14 | Laser 3 Macro | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH15 | Laser 4 Macro | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH16 | Laser 5 Macro | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH17 | Laser 6 Macro | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH18 | Reset / Auto | 000-008 | NO FUNCTION |
|  |  | 009-019 | SHOW1 |
|  |  | 020-030 | SHOW2） |
|  |  | 031-041 | SHOW3 |
|  |  | 042-052 | SHOW4 |
|  |  | 053-063 | SHOW5 |
|  |  | 064-074 | SHOW6 |
|  |  | 075-085 | SHOW7 |
|  |  | 086-096 | SHOW8 |
|  |  | 097-107 | SHOW9 |
|  |  | 108-118 | SHOW10 |
|  |  | 119-129 | SHOW11 |
|  |  | 130-140 | SHOW12 |
|  |  | 141-151 | SHOW13 |
|  |  | 152-162 | SHOW14 |
|  |  | 163-173 | SHOW15 |
|  |  | 174-184 | SHOW16 |
|  |  | 185-195 | SHOW17 |
|  |  | 196-206 | SHOW18 |
|  |  | 207-217 | SHOW19 |
|  |  | 218-228 | SHOW20 |
|  |  | 229-239 | SHOW21 |
|  |  | 240-250 | Auto effect |
|  |  | 251-255 | Reset |
| CH19 | Auto program / Sound mode | 000-250 | Program speed from slow to fast |
|  |  | 251-255 | Sound mode |

- 19-2 DMX Channel chart

| Channel |  | Value | Function |  |
| --- | --- | --- | --- | --- |
| CH1 | Tilt1 | 000-255 | Tilt angle：0-90° |  |
| CH2 | Laser 1 Macro | 000-010 | OFF |  |
|  |  | 011-045 | R |  |
|  |  | 046-080 | G |  |
|  |  | 081-115 | B |  |
|  |  | 116-150 | R&G |  |
|  |  | 151-185 | R&B |  |
|  |  | 186-220 | G&B |  |
|  |  | 221-255 | R&G&B |  |
| CH3 | Tilt2 | 000-255 | Tilt angle：0-90° |  |
| CH4 | Laser 2 Macro | 000-010 | OFF |  |
|  |  | 011-045 | R |  |
|  |  | 046-080 | G |  |
|  |  | 081-115 | B |  |
|  |  | 116-150 | R&G |  |
|  |  | 151-185 | R&B |  |
|  |  | 186-220 | G&B |  |
|  |  | 221-255 | R&G&B |  |
| CH5 | Tilt3 | 000-255 | Tilt angle：0-90° |  |
| CH6 | Laser 3 Macro | 000-010 | OFF |  |
|  |  | 011-045 | R |  |
|  |  | 046-080 | G |  |
|  |  | 081-115 | B |  |
|  |  | 116-150 | R&G |  |
|  |  | 151-185 | R&B |  |
|  |  | 186-220 | G&B |  |
|  |  | 221-255 | R&G&B |  |
| CH7 | Tilt4 | 000-255 | Tilt angle：0-90° |  |
| CH8 | Laser 4 Macro | 000-010 | OFF |  |
|  |  | 011-045 | R |  |
|  |  | 046-080 | G |  |
|  |  | 081-115 | B |  |
|  |  | 116-150 | R&G |  |
|  |  | 151-185 | R&B |  |
|  |  | 186-220 | G&B |  |
|  |  | 221-255 | R&G&B |  |
| CH9 | Tilt5 | 000-015 | Tilt angle：0-90° |  |
| CH10 | Laser 5 Macro | 000-010 | OFF |  |
|  |  | 011-045 | R |  |
|  |  | 046-080 | G |  |
|  |  | 081-115 | B |  |
|  |  | 116-150 | R&G |  |
|  |  | 151-185 | R&B |  |
|  |  | 186-220 | G&B |  |
|  |  | 221-255 | R&G&B |  |
| CH11 | Tilt6 | 000-255 | Tilt angle：0-90° |  |
| CH12 | Laser 6 Macro | 000-010 | OFF |  |
|  |  | 011-045 | R |  |
|  |  | 046-080 | G |  |
|  |  | 081-115 | B |  |
|  |  | 116-150 | R&G |  |
|  |  | 151-185 | R&B |  |
|  |  | 186-220 | G&B |  |
|  |  | 221-255 | R&G&B |  |
| CH13 | Dimmer | 000-127 | OFF |  |
|  |  | 128-255 | ON |  |
| CH14 | Strobe | 000-015 | OFF |  |
|  |  | 016-030 | ON |  |
|  |  | 031-090 | VARIABLE STROBE SLOW TO FAST |  |
|  |  | 091-105 | ON |  |
|  |  | 106-165 | PULSE EFFECT SLOW TO FAST |  |
|  |  | 166-180 | ON |  |
|  |  | 181-240 | RANDOM STROBE SLOW TO FAST |  |
|  |  | 241-255 | ON |  |
| CH15 | Pan | 000-255 | Pan angle：0-260° |  |
| CH16 | Pan Speed | 000-255 | Pan speed from fast to slow |  |
| CH17 | Tilt Speed | 000-255 | Pan speed from fast to slow |  |
| CH18 | Reset / Auto | 000-008 | NO FUNCTION |  |
|  |  | 009-019 | SHOW1 |  |
|  |  | 020-030 | SHOW2 |  |
|  |  | 031-041 | SHOW3 |  |
|  |  | 042-052 | SHOW4 |  |
|  |  | 053-063 | SHOW5 |  |
|  |  | 064-074 | SHOW6 |  |
|  |  | 075-085 | SHOW7 |  |
|  |  | 086-096 | SHOW8 |  |
|  |  | 097-107 | SHOW9 |  |
|  |  | 108-118 | SHOW10 |  |
|  |  | 119-129 | SHOW11 |  |
|  |  | 130-140 | SHOW12 |  |
|  |  | 141-151 | SHOW13 |  |
|  |  | 152-162 | SHOW14 |  |
|  |  | 163-173 | SHOW15 |  |
|  |  | 174-184 | SHOW16 |  |
|  |  | 185-195 | SHOW17 |  |
|  |  | 196-206 | SHOW18 |  |
|  |  | 207-217 | SHOW19 |  |
|  |  | 218-228 | SHOW20 |  |
|  |  | 229-239 | SHOW21 |  |
|  |  | 240-250 | Auto effect |  |
|  |  | 251-255 | Reset |  |
| CH19 | Auto program / Sound mode | 000-250 | Program speed from slow to fast |  |
|  |  | 251-255 | Sound mode |  |

- 32 DMX Channel chart

| Channel |  | Value | Function |
| --- | --- | --- | --- |
| CH1 | Pan | 000-255 | Pan angle：0-260° |
| CH2 | PanSpeed | 000-255 | Pan speed from fast to slow |
| CH3 | Tilt1 | 000-255 | Tilt angle：0-90° |
| CH4 | Tilt2 | 000-255 | Tilt angle：0-90° |
| CH5 | Tilt3 | 000-255 | Tilt angle：0-90° |
| CH6 | Tilt4 | 000-255 | Tilt angle：0-90° |
| CH7 | Tilt5 | 000-015 | Tilt angle：0-90° |
| CH8 | Tilt6 | 000-255 | Tilt angle：0-90° |
| CH9 | TiltSpeed | 000-255 | Tilt speed from fast to slow |
| CH10 | Dimmer | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH11 | Strobe | 000-015 | OFF |
|  |  | 016-030 | ON |
|  |  | 031-090 | VARIABLE STROBE SLOW TO FAST |
|  |  | 091-105 | ON |
|  |  | 106-165 | PULSE EFFECT SLOW TO FAST |
|  |  | 166-180 | ON |
|  |  | 181-240 | RANDOM STROBE SLOW TO FAST |
|  |  | 241-255 | ON |
| CH12 | Laser-R1 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH13 | Laser-G1 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH14 | Laser-B1 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH15 | Laser-R2 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH16 | Laser-G2 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH17 | Laser-B2 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH18 | Laser-R3 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH19 | Laser-G3 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH20 | Laser-B3 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH21 | Laser-R4 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH22 | Laser-G4 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH23 | Laser-B4 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH24 | Laser-R5 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH25 | Laser-G5 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH26 | Laser-B5 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH27 | Laser-R6 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH28 | Laser-G6 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH29 | Laser-B6 | 000-127 | OFF |
|  |  | 128-255 | ON |
| CH30 | LaserMacro | 000-010 | OFF |
|  |  | 011-045 | R |
|  |  | 046-080 | G |
|  |  | 081-115 | B |
|  |  | 116-150 | R&G |
|  |  | 151-185 | R&B |
|  |  | 186-220 | G&B |
|  |  | 221-255 | R&G&B |
| CH31 | Reset / Auto | 000-008 | NO FUNCTION |
|  |  | 009-019 | SHOW1 |
|  |  | 020-030 | SHOW2 |
|  |  | 031-041 | SHOW3 |
|  |  | 042-052 | SHOW4 |
|  |  | 053-063 | SHOW5 |
|  |  | 064-074 | SHOW6 |
|  |  | 075-085 | SHOW7 |
|  |  | 086-096 | SHOW8 |
|  |  | 097-107 | SHOW9 |
|  |  | 108-118 | SHOW10 |
|  |  | 119-129 | SHOW11 |
|  |  | 130-140 | SHOW12 |
|  |  | 141-151 | SHOW13 |
|  |  | 152-162 | SHOW14 |
|  |  | 163-173 | SHOW15 |
|  |  | 174-184 | SHOW16 |
|  |  | 185-195 | SHOW17 |
|  |  | 196-206 | SHOW18 |
|  |  | 207-217 | SHOW19 |
|  |  | 218-228 | SHOW20 |
|  |  | 229-239 | SHOW21 |
|  |  | 241-250 | Auto effect |
|  |  | 251-255 | Reset |
| CH32 | Auto program / Sound mode | 000-250 | Program from speed slow to fast |
|  |  | 251-255 | Sound mode |

DMX-512 connection

Connect the provided XLR cable to the male 3-pin XLR output of your controller and the other side to the female 3-pin XLR input of the light.You can chain multiple lights together through serial linking

- The cable needed should be two core, screened cable with XLR input and output connectors. Please refer to the diagram below.

XLR Mounting socket

XLR Mounting plug

DMX address setting

Each fixture must have a specific starting address. When receiving signal transmission, fixture will receive channel control signals from DMX controller through this starting address.

According to different requirements or easy to use, the user can set many fixtures to the same address, also can set up a separately address for every device.

If more than one fixtures have the same address, all of them receive the same DMX channel signal from DMX controller. All connected fixtures are jointly controlled, DMX controller can't control a device separately.

If each device has a different address, each device will be separate and accept separate DMX512 control signal through the separate DMX address, so that more convenient to control one device separately. this method must be based on channel quantity of device to determine a starting address.

This device has 55 channels, so the first device starting address set 1, second is 56(1+55), third is 111(56+55) and so on.

Troubleshooting

Following are a few common problems that may occur during operation. Here are some suggestions for easy troubleshooting:

- The fixture does not work, no light

- 1. Check the connection of power and main fuse.

- 2. Measure the mains voltage on the main connector.

- B. Not responding to DMX controller

- 1. DMX LED should be on. If not, check DMX connectors, cables to see if link properly.

- 2. If the DMX LED is on and no response to the channel, check the address settings and DMX polarity.

- 3. If you have intermittent DMX signal problems, check the pins on connectors or on PCB of the fixture or the previous one.

- 4. Try to use another DMX controller.

- 5. Check if the DMX cables run near or run alongside to high voltage cables that may cause damage or interference to DMX interface circuit.

Fixture Cleaning

- The cleaning of internal must be carried out periodically to optimize light output. Cleaning frequency depends on the environment in which the fixture operates: damp, smoky or particularly dirty surrounding can cause greater accumulation of dirt on the fixture’s optics.

- Clean with soft cloth using normal glass cleaning fluid.

- Always dry the parts carefully.

- Clean the external optics at least every 15 days. Clean the internal optics at least every 30/60 days.

Warranty Card

We will provide 1 year warranty under the condition that user has operated the light normally and lifetime service. We won't provide warranty if the damage is caused by artificial or force majeure event. Cost of fittings should be charged by user if product needs maintenance after 1 year .Please cut this warranty card and shipped it along with the product to our factory when applying for warranty.

Product： Purchasing date： Code of invoice：

Warranty date: From To

User’s name： Add：

Company： Tel：